174 Which two statements describe the process identifier that is used in the command to configure OSPF on a router? (Choose two.)

Router(config)# router ospf 1

A. All OSPF routers in an area must have the same process ID.

B. Only one process number can be used on the same router.

C. Different process identifiers can be used to run multiple OSPF processes

D. The process number can be any number from 1 to 65,535.

E. Hello packets are sent to each neighbor to determine the processor identifier.

Answer: C,D

200 A network administrator needs to configure port security on a switch. Which two statements are true? (Choose two.)

A. The network administrator can apply port security to dynamic access ports.

B. The network administrator can apply port security to EtherChannels.

C. When dynamic MAC address learning is enabled on an interface, the switch can learn new addresses, up to the maximum defined.

D. The sticky learning feature allows the addition of dynamically learned addresses to the running configuration.

E. The network administrator can configure static secure or sticky secure MAC addresses in the voice VLAN.

Answer: C,D

213

A network administrator is configuring ACLs on a Cisco router, to allow traffic from hosts on networks 192.168.146.0, 192.168.147.0, 192.168.148.0, and 192.168.149.0 only. Which two ACL statements, when combined, would you use to accomplish this task? (Choose two.)

A. access-list 10 permit ip 192.168.146.0 0.0.1.255

B. access-list 10 permit ip 192.168.147.0 0.0.255.255

C. access-list 10 permit ip 192.168.148.0 0.0.1.255

D. access-list 10 permit ip 192.168.149.0 0.0.255.255

E. access-list 10 permit ip 192.168.146.0 0.0.0.255

F. access-list 10 permit ip 192.168.146.0 255.255.255.0

Answer: A,C