QUESTION 31

What occurs on a Frame Relay network when the CIR is exceeded? (3.3.3-1)

A. All TCP traffic is marked discard eligible.

B. All UDP traffic is marked discard eligible and a BECN is sent.

C. All TCP traffic is marked discard eligible and a BECN is sent.

D. All traffic exceeding the CIR is marked discard eligible.

Answer: D

Explanation/Reference:

在FR網路中，當CIR超過時會發生什麼情況？

D：所有超過的流量都丟棄。

QUESTION 32

What are two characteristics of Frame Relay point-to-point subinterfaces? (Choose two.) (3.3.1-2)

A. They create split-horizon issues.

B. They require a unique subnet within a routing domain.

C. They emulate leased lines.

D. They are ideal for full-mesh topologies.

E. They require the use of NBMA options when using OSPF.

http://certiking.blogspot.com/

Answer: BC

Explanation/Reference:

幀中繼點到點子介面的特點？

A：水準分割，錯誤。

B：每個子介面需要一個獨立的網段，正確。

C：類似專線，正確。

D:全網互聯是理想拓撲，錯誤。通常是hub-spoke

E：NBMA不支援OSPF，錯誤。

QUESTION 33

Refer to the exhibit. Addresses within the range 10.10.10.0/24 are not being translated to the 1.128.0/16 range. Which command shows if 10.10.10.0/24 are allowed inside addresses? (5.4.5-1)

A. debug ip nat

B. show access-list

C. show ip nat translation

D. show ip nat statistics

Answer: B

Explanation/Reference:

那條命令可以看到10.10.10.0/24是否在允許轉換範圍

。允許的範圍是通過ACL定義的。

A：打開NAT轉換的debug資訊，錯誤

B：查看acl資訊，正確。

C：查看NAT轉換表，錯誤。

D：查看靜態NAT。錯誤

QUESTION 34

A wireless client cannot connect to an 802.11b/g BSS with a b/g wireless card. The client section of the access point does not list any active WLAN clients. What is a possible reason for this?  (7.4.1-1)

A. The incorrect channel is configured on the client.

B. The client's IP address is on the wrong subnet.

C. The client has an incorrect pre-shared key.

D. The SSID is configured incorrectly on the client.

Answer: D

Explanation/Reference:

用戶端無法連接到無限接入點，查看不到可用的無線信號。這通常是SSID信號沒有配置正確。

A：不正確的通道。錯誤

B：用戶端的IP位址配置錯誤

C：用戶端的共用key錯誤。

D：SSID配置不正確。

QUESTION 35

Which two features did WPAv1 add to address the inherent weaknesses found in WEP? (Choose two.) (7.2.2-1)

http://certiking.blogspot.com/

A. a stronger encryption algorithm

B. key mixing using temporal keys

C. shared key authentication

D. a shorter initialization vector

E. per frame sequence counters

Answer: BE

Explanation/Reference:

考點：WPA1中增強哪兩個在WEP中發現的弱點？WEP存在的隱患主要是依賴共用密鑰

A：更強的加密演算法

B：混合使用的密鑰

C：共用密鑰認證

D：較短的初始化向量

E：一個幀一個序列計數器。

QUESTION 36

Which two wireless encryption methods are based on the RC4 encryption algorithm? (Choose two.) (7.2.2-3)

A. WEP

B. CCKM

C. AES

D. TKIP

E. CCMP

Answer: AD

Explanation/Reference:

考點：哪兩種無限加密技術依賴RC4

A:有線等效保密協議

B：英代爾開發一種協議

C：高級加密標準

D：暫時密鑰集成協議

E:基於AES的加密方式

QUESTION 37

What are two characteristics of RIPv2? (Choose two.) (7.2.1-1)

A. classful routing protocol

B. variable-length subnet masks

C. broadcast addressing

D. manual route summarization

E. uses SPF algorithm to compute path

Answer: BD

Explanation/Reference:

A：有類路由協定，錯誤

B：可變長度子網路遮罩，正確

C：採用廣播位址更新，錯誤

D：手動路由匯總，正確

E:使用spf演算法計算路徑，錯誤。

QUESTION 38

Which two Ethernet fiber-optic modes support distances of greater than 550 meters?

(8.3.1-1)

A. 1000BASE-CX

B. 100BASE-FX

C. 1000BASE-LX

D. 1000BASE-SX

E. 1000BASE-ZX

Answer: CE

Explanation/Reference:

A：最長距離25米

B：最長距離100米

C：最長距離3000米

D：最長距離550米

E：最長距離70千米

QUESTION 39

What two things will a router do when running a distance vector routing protocol? (Choose two.) (4.1.2-2)

A. Send periodic updates regardless of topology changes.

B. Send entire routing table to all routers in the routing domain.

C. Use the shortest-path algorithm to the determine best path.

D. Update the routing table based on updates from their neighbors.

E. Maintain the topology of the entire network in its database.

Answer: AD

Explanation/Reference:

考點：考查距離向量路由協議特性

A：週期性更新，正確

B：發送整個路由表到整個路由域，錯誤，只發送給鄰居

C：使用SPF演算法，錯誤

D：依靠鄰居發送更新來更新路由表，正確

E：在資料庫中維護整個網路的拓撲。錯誤

QUESTION 40

Refer to the exhibit. According to the routing table, where will the router send a packet destined for 10.1.5.65? (8.2.2-1)

A. 10.1.1.2

B. 10.1.2.2

C. 10.1.3.3

D. 10.1.4.4

Answer: C

Explanation/Reference:

路由查找原則：最長遮罩匹配原則。

QUESTION 41

Which command shows if an access list is assigned to an interface? (11.3.2-1)

A. show ip interface [interface] access-lists

B. show ip access-lists interface [interface]

C. show ip interface [interface]

D. show ip access-lists [interface]

Answer: C

Explanation/Reference:

QUESTION 42

Refer to the exhibit. Which rule does the DHCP server use when there is an IP address conflict? (7.1.8-1)

A. The address is removed from the pool until the conflict is resolved.

B. The address remains in the pool until the conflict is resolved.

C. Only the IP detected by Gratuitous ARP is removed from the pool.

D. Only the IP detected by Ping is removed from the pool.

E. The IP will be shown, even after the conflict is resolved.

Answer: A

Explanation/Reference:

show ip dhcp conflict命令可以查看哪些ip位址在使用上有衝突。

有衝突的地址不再分配出去。

A：地址被刪除直到衝突解決，正確

B:位址保留直到衝突解決，錯誤

C/D：答案片面。

E：當衝突解決後，位址會被顯示。錯誤。

QUESTION 43

Refer to the exhibit. You are connected to the router as user Mike. Which command allows you to see output from the OSPF debug command? (4.5.6-5)

A. terminal monitor

B. show debugging

C. show sessions

D. show ip ospf interface

Answer: A

Explanation/Reference:

A：讓debug資訊在終端上顯示出來

B：沒這條命令

C：查看會話信息。

D:查看啟用了ospf的介面的相關資訊。

QUESTION 44

Refer to the exhibit. If number 2 is selected from the setup script, what happens when the user runs setup from a privileged prompt? (11.1.3-3)

A. Setup is additive and any changes will be added to the config script.

B. Setup effectively starts the configuration over as if the router was booted for the first time.

C. Setup will not run if an enable secret password exists on the router.

D. Setup will not run, because it is only viable when no configuration exists on the router.

Answer: A

Explanation/Reference:

[0]不保存進入IOS命令配置模式

[1]不保存返回設置模式

[2]保存退出

A：設置和改動被保存到配置腳本

B：為有效地重新開始配置，如果路由器是第一次啟動的配置。

C：如果配置了密碼，設置不會運行

D：設置不可用，因為它只能在沒有配置存在時才有效。

QUESTION 45

Refer to the exhibit. Which (config-router) command will allow the network represented on the interface to be advertised by RIP? (5.2.3-1)

A. redistribute ethernet0

B. network ethernet0

C. redistribute 10.12.0.0

D. network 10.12.0.0

Answer: D

Explanation/Reference:

考點：怎麼才能讓介面通告RIP路由。

通告network網段實現

QUESTION 46

Refer to the exhibit. What information can be gathered from the output? (5.3.3-1)

A. One router is running RIPv1.

B. RIP neighbor is 224.0.0.9.

C. The network contains a loop.

D. Network 10.10.1.0 is reachable.

Answer: D

Explanation/Reference:

由圖中可以看出本地已經存在10.10.1.0這條路由

QUESTION 47

Refer to the exhibit. What type of connection would be supported by the cable diagram shown? (8.3.3-2)

A. PC to router

B. PC to switch

C. server to router

D. router to router

Answer: B

Explanation/Reference:

從圖中可以看出這是一條直通線。同構設備使用交叉線，異構設備使用直通線。

B：直通線

AC/D同構設備使用交叉線

QUESTION 48

Refer to the exhibit. What type of connection would be supported by the cable diagram shown? (8.3.3-2)

A. PC to router

B. PC to switch

C. server to switch

D. switch to router

Answer: A

Explanation/Reference:

QUESTION 49

Which two statements describe the Cisco implementation of VLANs? (Choose two.)

(3.1.1-4)

A. VLAN 1 is the default Ethernet VLAN.

B. CDP advertisements are only sent on VLAN 1002.

C. By default, the management VLAN is VLAN 1005.

D. By default, the switch IP address is in VLAN 1005.

E. VLAN 1002 through 1005 are automatically created and cannot be deleted.

Answer: AE

Explanation/Reference:

考點：考查vlan知識點。

A：缺省都存在vlan1，埠都屬於vlan1，正確

B：CDP資訊只發送到vlan1002，錯誤，vlan1002是fddi網路的vlan號。

C：缺省情況交換機IP配置在vlan1005，錯誤，vlan1005trnet網路的vlan號。缺省在vlan1裏面。

D：vlan1002-1005自動生成並不能被刪除，正確，他們都是被保留的特殊vlan

QUESTION 50

Refer to the exhibit. What can be determined about the router from the console output? (11.2.3-1)(1.1.4-2)

。

A. No configuration file was found in NVRAM.

B. No configuration file was found in flash.

C. No configuration file was found in the PCMCIA card.

D. Configuration file is normal and will load in 15 seconds.

Answer: A

Explanation/Reference:

啟動時缺省讀取保存在NVRAM裏面的配置檔。提示進入對話模式，說明沒有保存配置檔。

QUESTION 51

Refer to the exhibit. What can be determined from the output? (11.4.3-1)

A. 192.168.1.2 is local to the router.

B. 192.168.3.1 is local to the router.

C. 192.168.1.2 will age out in less than 1 minute.

D. 192.168.3.1 has aged out and is marked for deletion.

Answer: B

Explanation/Reference:

age下面- 代表是本地埠

QUESTION 52

Refer to the exhibit. Which command would allow the translations to be created on the router? (7.2.5-1)

A. ip nat pool mynats 1.1.128.1 1.1.135.254 prefix-length 19

B. ip nat outside mynats 1.1.128.1 1.1.135.254 prefix-length 19

C. ip nat pool mynats 1.1.128.1 1.1.135.254 prefix-length 18

D. ip nat outside mynats 1.1.128.1 1.1.135.254 prefix-length 18

Answer: A

Explanation/Reference:

prefix-length 表示遮罩的長度。prefix-length 19表示遮罩為255.255.224.0

QUESTION 53

Refer to the exhibit. An administrator pings the default gateway at 10.10.10.1 and sees the output as hown. At which OSI layer is the problem? (6.6.5-1)

A. data link layer

B. application layer

C. access layer

D. session layer

E. network layer

Answer: E

Explanation/Reference:

ping調用icmp，ping不通是網路層的原因。

QUESTION 54

Refer to the exhibit. Which value will be configured for Default Gateway of the Local Area Connection? (6.3.2.1)

A. 10.0.0.0

B. 10.0.0.254

C. 192.223.129.0

D. 192.223.129.254

Answer: B

Explanation/Reference:

10.0.0.1--10.0.0.254裏面除了10.0.0.249外任意一個。

QUESTION 55

Refer to the exhibit. The user at Workstation B reports that Server A cannot be reached. What is peventing Workstation B from reaching Server A? (6.5.1-2)

A. The IP address for Server A is a broadcast address.

B. The IP address for Workstation B is a subnet address.

C. The gateway for Workstation B is not on the same subnet.

D. The gateway for Server A is not on the same subnet.

Answer: D

Explanation/Reference:

131.1.123.24/27所在網路範圍為：131.1.123.0-31/27。很明顯閘道131.1.123.33不再範圍內。

QUESTION 56

Refer to the exhibit. What does the (*) represent in the output? (7.2.8-2)

A. Packet is destined for a local interface to the router.

B. Packet was translated, but no response was received from the distant device.

C. Packet was not translated, because no additional ports are available.

D. Packet was translated and fast switched to the destination.

Answer: D

Explanation/Reference:

由圖中提示資訊可以看出，源為10.10.0.2已經有個一個轉換，所以後續的資料包快速轉發。

QUESTION 57

Refer to the exhibit. What command sequence will enable PAT from the inside to outside network? (7.2.6-2)

A. (config) ip nat pool isp-net 1.2.4.2 netmask 255.255.255.0 overload

B. (config-if) ip nat outside overload

C. (config) ip nat inside source list 1 interface ethernet1 overload

D. (config-if) ip nat inside overload

Answer: C

Explanation/Reference:

overload是配置PAT的關鍵字

QUESTION 58

Refer to the exhibit. What will happen to HTTP traffic coming from the Internet that is destined for 172.16.12.10 if the traffic is processed by this ACL? (5.1.4-1)

A. Traffic will be dropped per line 30 of the ACL.

B. Traffic will be accepted per line 40 of the ACL.

C. Traffic will be dropped, because of the implicit deny all at the end of the ACL.

D. Traffic will be accepted, because the source address is not covered by the ACL.

Answer: C

Explanation/Reference:

ACL匹配規則：

自上而下匹配，一旦匹配則終止，沒有匹配到的丟棄。

題目給的172是目的地址，表中ACL中172是源地址

QUESTION 59

Refer to the exhibit. Which statement describes the effect that the Router1 configuration has on devices in the 172.16.16.0 subnet when they try to connect to SVR-A using Telnet or SSH? (5.2.4-1)

A. Devices will not be able to use Telnet or SSH.

B. Devices will be able to use SSH, but not Telnet.

C. Devices will be able to use Telnet, but not SSH.

D. Devices will be able to use Telnet and SSH.

Answer: B

Explanation/Reference:

題目考查的是ACL用法，首先我們看圖示

要控制的是172網段也就是F0/0 是in方向

f0/1 應該是out方向也就是從左到右的資料

通過此圖我們看出list 100 為fa0/0的in方向

而list101 是f0/1的in方向所以對172網段不會生效

生效的是list 100

我們只要看list 100 就可以選出答案可以SSH

而20格式不正確，應該將eq放在目的地址後面才會起作用。所以可以SSH但是不能telnet。

QUESTION 60

What are three advantages of VLANs? (Choose three.) (3.1.1-3)

A. VLANs establish broadcast domains in switched networks.

B. VLANs utilize packet filtering to enhance network security.

C. VLANs provide a method of conserving IP addresses in large networks.

D. VLANs provide a low-latency internetworking alternative to routed networks.

E. VLANs allow access to network services based on department, not physical location.

F. VLANs can greatly simplify adding, moving, or changing hosts on the network.

Answer: AEF

Explanation/Reference:

A：VLAN在交換網路中建立廣播域，正確

B：利用VLAN的資料包過濾，增強網路的安全性。錯誤，沒有這個特性。

C：提供了在大型網路中的保護IP位址的方法。錯誤。

D：提供低延遲互聯網路替代路由網路。錯誤

E：vlan允許基於邏輯劃分的網路訪問，不是物理位置。正確。

F：VLAN可以大大簡化添加，移動或更改網路上的主機。正確。

QUESTION 61

An administrator would like to configure a switch over a virtual terminal connection from locations outside of the local LAN. Which of the following are required in order for the switch to be configured from a remote location? (Choose two.) (10.3.3-1)

A. The switch must be configured with an IP address, subnet mask, and default gateway.

B. The switch must be connected to a router over a VLAN trunk.

C. The switch must be reachable through a port connected to its management VLAN.

D. The switch console port must be connected to the Ethernet LAN.

E. The switch management VLAN must be created and have a membership of at least one switch port.

F. The switch must be fully configured as an SNMP agent.

Answer: AC

Explanation/Reference:

要想遠端管理VLAN，1、交換機配置IP，2、路由可達。

QUESTION 62

Which of the following host addresses are members of networks that can be routed across the public Internet? (Choose three.) (6.2.5-1)

A. 10.172.13.65

B. 172.16.223.125

C. 172.64.12.29

D. 192.168.23.252

E. 198.234.12.95

F. 212.193.48.254

Answer: CEF

Explanation/Reference:

考查公有IP地址的範圍。

10.0.0.0/8-10.255.255.255/8

172.16.0.0/16--172.31.255.255/16

192.168.0.0/24-192.168.255.255/24

除了上面的外都是公有IP。

QUESTION 63

Given a subnet mask of 255.255.255.224, which of the following addresses can be assigned to network hosts? (Choose three.) (6.5.1-1)

A. 15.234.118.63

B. 92.11.178.93

C. 134.178.18.56

D. 192.168.16.87

E. 201.45.116.159

F. 217.63.12.192

Answer: BCD

Explanation/Reference:

QUESTION 64

Which of the following are benefits of VLANs? (Choose three.) (3.1.1-3)

A. They increase the size of collision domains.

B. They allow logical grouping of users by function.

C. They can enhance network security.

D. They increase the size of broadcast domains while decreasing the number of collision domains.

E. They increase the number of broadcast domains while decreasing the size of the broadcast domains.

F. They simplify switch administration.

Answer: BCE

Explanation/Reference:

考查VLAN的特點

A：增加衝突域的大小，錯誤

B：允許邏輯劃分網段，正確

C：增強網路安全，正確。不同vlan不能直接通信。

D：當減少衝突域時增加廣播域的大小。錯誤

E：減少廣播域的大小時增加了廣播域個數。正確。

F:簡化交換機管理。答非所問。

QUESTION 65

Which router IOS commands can be used to troubleshoot LAN connectivity problems? (Choose three.) (6.6.3-1) (11.1.6-2)(2.7.2-1)

A. ping

B. tracert

C. ipconfig

D. show ip route

E. winipcfg

F. show interfaces

Answer: ADF

Explanation/Reference:

A：測試連通性

D：查看是否有路由

F：查看介面是否可用

QUESTION 66

Refer to the exhibit. After HostA pings HostB, which entry will be in the ARP cache of HostA to support this transmission?(9.7.2-1)

A.

B.

C.

D.

E.

F.

Answer: D

Explanation/Reference:

在乙太網定址的時候，依靠的事MAC位址，每一個網段都需要尋找到自己的下一條（gateway）以自己的MAC作為source gateway的mac作為destination封裝Frame，然後將Frame轉發出去，但是資料包是不會進行變化的，也就是packet的頭部資訊不會發生改變。所以IP包頭的source和des位址是不會變更的以自己的MAC作為source gateway的mac作為destination封裝Frame，然後將Frame轉發出去，

但是資料包是不會進行變化的，也就是packet的頭部資訊不會發生改變。所以IP包頭的source和des位址是不會變更的

QUESTION 67

Which two topologies are using the correct type of twisted-pair cables? (Choose two.)

(8.3.3-2)

A.

B.

C.

D.

E.

Answer: DE

Explanation/Reference:

線纜問題，使用雙絞線的設備。相同設備用crossover不同設備用cut-thought線纜。console使用的事rollor線纜

QUESTION 68

Which of the following are true regarding bridges and switches? (Choose two.)(2.1.2-5)(2.1.1-5)

A. Bridges are faster than switches because they have fewer ports.

B. A switch is a multiport bridge.

C. Bridges and switches learn MAC addresses by examining the source MAC address of each frame received.

D. A bridge will forward a broadcast but a switch will not.

E. Bridges and switches increase the size of a collision domain.

Answer: BC

Explanation/Reference:

考查網橋和交換機的區別

A：網橋比交換機更快速，因為他們有更少的埠。錯誤

B：交換機是一個多埠網橋。正確

C：通過檢查資料包的source mac學習mac位址，正確

D：網橋轉發廣播包，但交換機不轉發。錯誤

E：網橋和交換機增加衝突域的大小，錯誤。

QUESTION 69

What are some of the advantages of using a router to segment the network? (Choose two.)(2.1.2-2)

A. Filtering can occur based on Layer 3 information.

B. Broadcasts are eliminated.

C. Routers generally cost less than switches.

D. Broadcasts are not forwarded across the router.

E. Adding a router to the network decreases latency.

Answer: AD

Explanation/Reference:

A：可以基於層三的資訊過濾資料。

B：廣播被淘汰。錯誤

C：路由器的價格一般比交換機少。錯誤

D：廣播不會穿越路由器，正確。

E:增加路由器後減少延遲。錯誤。

QUESTION 70

Which of the following statements are true regarding bridges and switches? (Choose 3.)(2.1.2-5)(2.1.2-2)

A. Switches are primarily software based while bridges are hardware based.

B. Both bridges and switches forward Layer 2 broadcasts.

C. Bridges are frequently faster than switches.

D. Switches have a higher number of ports than most bridges.

E. Bridges define broadcast domains while switches define collision domains.

F. Both bridges and switches make forwarding decisions based on Layer 2 addresses.

Answer: BDF

Explanation/Reference:

A：交換機基於軟體，網橋基於硬體，錯誤。都是基於硬體

B：都轉發二層廣播包，正確

C：網橋比交換機快速。錯誤。

D：交換機比網橋具有更多的埠，正確。

E：網橋定義廣播域，交換機定義衝突域，錯誤。

F:基於二層位址轉發資料。正確