QUESTION 151

Refer to the topology and router output shown in the exhibit. A technician is troubleshooting host connectivity issues on the switches. The hosts in VLANs 10 and 15 on Sw11 are unable to communicate with hosts in the same VLANs on Sw12. Hosts in the Admin VLAN are able to communicate. The port-to-VLAN assignments are identical on the two switches. What could be the problem?(6.3.1-1)

A. The Fa0/1 port is not operational on one of the switches.

B. The Link connecting the switches has not been configured as a trunk.

C. At lease one port needs to be configured in VLAN 1 for VLANs 10 and 15 to be able to communicate.

D. Port FastEthernet 0/1 needs to be configured as an access link on both switches.

E. A router is required for hosts on Sw11 in VLANs 10 and 15 to communicate with hosts in the same

VLAN on Sw12.

Answer: B

Explanation/Reference:

fa0/1. 此介面在一個vlan下面.所以介面並不是trunk

QUESTION 152

You have finished physically installing an access point on the ceiling at your office. At a minimum, which paramenter must be configured on the access point in order to allow a wireless client to operate on it?(7.1.4-3)

A. AES

B. PSK

C. SSID

D. TKIP

E. WEP

Answer: C

Explanation/Reference:

QUESTION 153

Refer to the exhibit. A network administrator needs to add a new VLAN, named VLAN3, to the network shown. Unfortunately, there is not another FastEthernet interface on R1 to connect to the new VLAN3. Which approach is the most cost effective solution for this problem?(6.1.2-5)

A. Purchase a new FastEthernet module and install it on R1

B. Replace R1 with a new router that has at least three FastEthernet interfaces.

C. Configure a second switch to support VLAN3 with a VLAN trunk between SW1 and the new switch.

D. Configure a single VLAN trunk between R1 and SW1 and configure subinterface on R1 interface for each VLAN.

E. Connect another router to a serial interface of R1. Use a FastEthernet interface on the new router for VLAN3.

Answer: D

Explanation/Reference:

考查單臂路由的基本配置,在配置單臂路由的時候.我們要針對每一個需要路由的vlan建立一個子介面.並且配上IP地址.此ip即為host的閘道

QUESTION 154

Refer to the exhibit. When running EIGRP, What is required for RouterA to exchange routing updates with RouterC?(9.7.1-1)

A. As numbers must be changed to match on all the routers.

B. Loopback interface must be configured so a DR is elected.

C. The no auto-summary command is needed on Router A and Router C.

D. Router B needs to have two network statements, one for each connected network.

Answer: A

Explanation/Reference:

eigrp的AS號碼必須一致.負責將不能夠形成鄰居關係所以也就不能夠進行路由update

QUESTION 155

Refer to the exhibit. The two exhibit devices are the only Cisco devices on the network. The serial network between the two devices has a mask of 255.255.255.252. Given the output that is shown, what three statements are true of these devices? (Choose three.)(2.3.3-2)

A. The Manchester serial address is 10.1.1.1

B. The Manchester serial address is 10.1.1.2

C. The London router is a Cisco 2610.

D. The Manchester router is a Cisco 2610.

E. The CDP information was received on port Serial0/0 of the Manchester router.

F. The CDP information was sent by port Serial0/0 of the London Router.

Answer: ACE

Explanation/Reference:

10.1.1.2-對端IP,因為遮罩為255.255.255.252所以自己可用IP則為10.1.1.1

interface:前面是自己的埠，後面是鄰居的埠

QUESTION 156

How is an EUI-64 format interface ID created from a 48-bit MAC address?(7.3.2-3)

A. by appending 0xFF to the MAC address.

B. by prefixing the MAC address with 0xFFEE.

C. by prefixing the MAC address with 0xFF and appending 0xFF to it.

D. by inserting 0xFFFE between the upper three bytes and the lower three bytes of the MAC address

E. by prefixing the MAC address with 0xF and inserting 0xF after each of its first three bytes.

Answer: D

Explanation/Reference:

QUESTION 157

Refer to the exhibit. After a RIP route is marked invalid on Router_1, how much time will elapse before that route is removed from the routing table?(4.3.1-2)

A. 30 seconds

B. 60 seconds

C. 90 seconds

D. 180 seconds

E. 240 seconds

Answer: B

Explanation/Reference:

RIP如果路由失效,holdtime之內都會表示passible down,經過60S後刷新時間才會從路由表刪除

QUESTION 158

Refer to the exhibit. A network associate has configured the internetwork that is shown in the exhibit, but has failed to configure routing properly.(2.4.2-1)

Which configuration will allow the hosts on the Branch LAN to access resources on the HQ LAN with the

least impact on router processing and WAN bandwidth?

A. HQ(config)# ip route 192.168.1.0 255.255.255.0 192.168.2.5

Branch(config)# ip route 172.16.25.0 255.255.255.0 192.168.2.6

B. HQ(config)# router rip

HQ(config-router)# network 192.168.2.0

HQ(config-router)# network 172.16.0.0

Branch(config)# router rip

Branch(config-router)# network 192.168.1.0

Branch(config-router)# network 192.168.2.0

C. HQ(config)# router eigrp 56

HQ(config-router)# network 192.168.2.4

HQ(config-router)# network 172.16.25.0

Branch(config)# router eigrp 56

Branch(config-router)# network 192.168.1.0

Branch(config-router)# network 192.168.2.4

D. HQ(config)# router ospf 1

HQ(config-router)# network 192.168.2.4 0.0.0.3 area 0

HQ(config-router)# network 172.16.25.0 0.0.0.255 area 0

Branch(config)# router ospf 1

Branch(config-router)# network 192.168.1.0 0.0.0.255 area 0

Answer: A

Explanation/Reference:

QUESTION 159

Which additional configuration step is necessary in order to connect to an access point that has SSID broadcasting disabled?(7.2.3-1)

A. Set the SSID value in the client software to public.

B. Configure open authentication on the AP and the client.

C. Set the SSID value on the client to the SSID configured on the AP.

D. Configured MAC address filtering to permit the client to connect to the AP.

Answer: C

Explanation/Reference:

QUESTION 160

What is one reason that WPA encryption is preferred over WEP?(7.2.2-1)

A. A WPA key is longer and requires more special characters than the WEP key.

B. The access point and the client are manually configured with different WPA key values.

C. WPA key values remain the same until the client configuration is changed.

D. The values of WPA keys can change dynamically while the system is used.

Answer: D

Explanation/Reference:

QUESTION 161

A network administrator is troubleshooing an EIGRP problem on a router and needs to confirm the IP addresses of the devices with which the router has established adjacency. The retransmit interval and the queue counts for the adjacent routers also need to be checked. What command will display the required information?(9.2.5-1)

A. Router# show ip eigrp adjacency

B. Router# show ip eigrp topology

C. Router# show ip eigrp interfaces

D. Router# show ip eigrp neighbors

Answer: D

Explanation/Reference:

問的是,需要確認哪些建立了鄰接關係.並且確認鄰接的參數,我們使用show ip eigrp neighbors就能查看得到了

QUESTION 162

All WAN links inside the ABC University network use PPP with CHAP for authentication security. Which command will display the CHAP authentication process as it occur between two routers in the network?(2.4.6-1)

A. show chap authentication

B. show interface serial0

C. debug ppp authentication

D. debug chap authentication

E. show ppp authentication chap

Answer: C

Explanation/Reference:

查看ppp認證的命令可以使用debug查看到ppp認證的過程.

QUESTION 163

Refer to the exhibit. A network technician is asked to design a small network with redundancy. The exhibit represents this design, with all hosts configured in the same VLAN. What conclusions can be made about this design?(6.4.2-1)

A. The design will function as intended

B. Spanning-tree will need to be used.

C. The router will not accept the addressing scheme.

D. The connection between switches should be a trunk.

E. The router interfaces must be encapsulated with the 802.1Q protocol.

Answer: C

Explanation/Reference:

路由器的介面上不會出現同一網段的兩個IP位址，這樣的IP位址是配置不上去的

針對這麼一個網路設計.路由器將不會提供備份的功能.因為在下面主機所致的閘道只有一個.,要麼1.1

要麼1.2

如果鏈路出現問題將不能夠進行切換

QUESTION 164

Refer to the exhibit. How should the FastEthernet0/1 port on the 2950 model switches that are shown in the exhibit be configured to allow connectivity between all devices?

(6.3.1-1)

A. The ports only need to be connected by a crossover cable.

B. SwitchX(config)# interface FastEthernet 0/1

SwitchX(config-if)# switchport mode trunk

C. SwitchX(config)# interface FastEthernet 0/1

SwitchX(config-if)# switchport mode access

SwitchX(config-if)# switchport access vlan 1

D. SwitchX(config)# interface FastEthernet 0/1

SwitchX(config-if)# switchport mode trunk

SwitchX(config-if)# switchport trunk vlan 1

SwitchX(config-if)# switchport trunk vlan 10

SwitchX(config-if)# switchport trunk vlan 20

Answer: B

Explanation/Reference:

fa0/0,fa0/1--此兩介面都必須配置成trunk,才能達到vlan間通信

而且sw1和sw2間的鏈路必須trunk才能帶到相同vlan的通信

QUESTION 165

IP address and routing for the network are configured as shown in the exhibit. The network administrator issues the show ip eigrp neighbors command from Router1 and receives the output shown below the topology. Which statement is true?(9.2.5-1)

A. It is normal for Router1 to show one active neighbor at a time to prevent routing loops.

B. Routing is not completely configured on Router3.

C. The IP addresses are not configured properly on the Router1 and Router3 interfaces.

D. The no auto-summary command configured on the routers prevents Router1 and Router2 from forming a neighbor relationship.

Answer: B

Explanation/Reference:

R1沒有R3的鄰居,R3並沒有宣告192.168.3.0這個網段.所以R1是沒有R3的鄰居

QUESTION 166

Refer to the exhibit. After SwitchB was added to the network, VLAN connectivity problems started to occur. What caused this problem?(4.3.2-5)

A. Both switches are in sever mode in the same domain

B. The revision number of SwitchB was higher than the revision number of SwitchA

C. SwitchA was not rebooted prior to adding SwitchB to the network.

D. V2-mode is not enabled.

E. VTP pruning is not activated, so the new paths in the network have not been recalculated.

Answer: B

Explanation/Reference:

QUESTION 167

Refer to the exhibit. The output that is shown is generated at a switch. Which three of these statements are true? (Choose three.)(5.4.3-1)

A. All ports will be in a state of discarding, learning, or forwarding.

B. Thiry VLANs have been configured on this switch.

C. The bridge priority is lower than the default value for spanning tree.

D. All interfaces that are shown are on shared media.

E. All designated ports are in a forwarding state.

F. The switch must be the root bridge for all VLANs on this switch.

Answer: CEF

Explanation/Reference:

rstp的所有介面在收斂完成之前都將經歷discarding->learning->forwarding的狀態,並且rstp的默認優先順序應該為32768，所有的DP都將轉發資料

QUESTION 168

Refer to the exhibit. A network administrator attempts to ping Host1 from Host1 and receives the result that are shown. What is a possible problem?(11.3.4-2)

A. The link between Host1 and Switch1 is down.

B. TCP/IP is not functioning on Host1.

C. The link between Router1 and Router2 is down.

D. The default gateway on Host1 is incorrect.

E. Interface Fa0/0 on Router1 is shudown.

F. The link between Switch1 and Router1 is down.

Answer: C

Explanation/Reference:

10.1.1.1回應的不可達報文,所以我們可以知道報文在到達10.1.1.1之後就沒有傳遞下去.也就是10.1.1.1之後的鏈路就已經失效.

QUESTION 169

Refer to the exhibit. The Bigtime router is unable to authenticate to the Littletime router. What is the cause of the problem?(2.4.5-2)

A. The usernames are incorrectly configured on the two routers.

B. The passwords do not match on the two routers.

C. CHAP authentication cannot be used on a serial interface.

D. The routers cannot be connected from interface S0/0 to interface S0/0.

E. With CHAP authentication, one router must authenticate to another router. The routers cannot be

configured to authenticate to each other.

Answer: B

Explanation/Reference:

ppp在進行認證配置的時候.用戶名要是對端的用戶名.但是password要是兩邊共用的password!

QUESTION 170

S0/0 on R1 is configured as a multipoint interface to communicate with R2 and R3 in the hub-and-spoke Frame Relay topology shown in the exhibit. Originally, static routes were configured between these routers to successfully route traffic between the attached networks. What will need to be done in order to use RIPv2 in place of the static routes?(7.4.3-1)(4.4.5-1)

A. Configure the no ip subnet-zero command on R1, R2, and R3.

B. Dynamic routing protocols such as RIPv2 cannot be used across Frame Relay networks.

C. Configure the S0/0 interface on R1 as two subinterfaces and configure point-to-point links to R2 and R3.

D. Change the 172.16.2.0/25 and 172.16.2.128/25 subnetworks so that at least two bits are borrowed from the last octet.

E. Change the network address configuration to eliminate the discontiguous 172.16.2.0/25 and 172.16.2.128/25 subnetwork.

Answer: C

Explanation/Reference:

中心節點在ping兩邊沒有問題.因為rip中心節點可以收到來自兩邊的路由.但是因為水準分割的問題,中心節點不會講路由更新再從這個介面發送出去.解決此問題最好的辦法就是中心節點設置子介面對每一個分支機搆,這樣從一個子介面收到的路由更新可以從另外一個位元組口發送出去.保證了網路的連通性

QUESTION 171

Refer to the exhibit. A router interface is being configured for Frame Relay. However, as the exhibit shows, the router will not accept the command to configure the LMI type. What is the problem?(7.3.5-1)(3.2.1-2)

A. The interface does not support Frame Relay connections.

B. The interface does not have an IP address assigned to it yet.

C. The interface requires that the no shutdown command be configured first.

D. the interface requires that the encapsulation frame-relay command be configured first.

Answer: D

Explanation/Reference:

串口的默認封裝為hdlc,要先改變封裝類型然後在改變封裝的type,所以我悶在配置FR的時候先要進行FR的封裝然後才改變FR的type

QUESTION 172

Refer to the exhibit. Two buildings on the San Jose campus of a small company must be connected to use Ethernet with a bandwidth of at least 100 Mbps. The company is concerned about possible problems from voltage potential difference between the two buildings. Which media type should be used for the connection?(10.2.1-3)(8.3.6-3)

A. UTP cable

B. STP cable

C. Coaxial cable

D. Fiber optic cable

Answer: D

Explanation/Reference:

QUESTION 173

Users have been complaining that their Frame Relay connection to the corporate site is very slow. The network administrator suspects that the link is overloaded. Based on the partial output of the Router# show frame relay pvc command shown in the graphic, which output value indicates to the local router that traffic sent to the corporate site is experiencing congestion?(3.3.3-1)

A. DLCI = 100

B. last time PVC status changed 00:25:40

C. in BECN packets 192

D. in FECN packets 147

E. in DE packets 0

Answer: C

Explanation/Reference:

BECN為後向擁塞指示，FECN為前向擁塞指示

QUESTION 174

Refer to the exhibit. A router boots to the prompt shown in the exhibit. What does this signify, and how should the network administrator respond?(4.5.5-1)

A. This prompt signifies that the configuration file was not found in NVRAM. The network administrator should follow the prompts to enter a basic configuration.

B. The prompt signifies that the configuration file was not found in flash memory. The network administrator should use TFTP to transfer a configuration file to the router.

C. The prompt signifies that the IOS image in flash memory is invalid or corrupted. The network administrator should use TFTP to transfer an IOS image to the router.

D. The prompt signifies that the router could not authenticate the user. The network administrator should modify the IOS image and reboot the router.

Answer: C

Explanation/Reference:

當路由器進入了rommon模式之下.我們首先要做的就是給其灌入IOS使用TFTP

QUESTION 175

Refer to the exhibit. Give this output for SwitchC, what should the network administrator's next action be?(7.2.3-1)

A. Check the trunk encapsulation mode for SwitchC's fa0/1 port.

B. Check the duplex mode for SwitchC's fa0/1 port.

C. Check the duplex mode for SwitchA's fa0/2 port.

D. Check the trunk encapsulation mode for SwitchA's fa0/2 port.

Answer: C

Explanation/Reference:

出現錯誤報文,可能是因為雙工不匹配或者鏈路不穩定等因素造成的

QUESTION 176

Refer to the exhibit. Which of these statements correctly describes the state of the switch once the boot process has been completed?(3.1.4-1)

A. As FastEthernet0/12 will be the last to come up, it will not be blocked by STP.

B. Remote access management of this switch will not be possible without configuration change.

C. More VLANs will need to be created for this switch.

D. The switch will need a different IOS code in order to support VLANs and STP.

Answer: C

Explanation/Reference:

從圖中可以看出交換機上只有vlan1 而且已經被關閉，沒有vlan的交換機是不能工作的，所以首先是要建立vlan

QUESTION 177

Refer to the exhibit. Which of these statements correctly describes the state of the switch once the boot process has been completed?(4.2.2-2)

A. Only the default VLANs are configured on SwitchA.

B. SwitchA does not have a VTP domain name configured.

C. VTP pruning needs to be enabled on SwitchA.

D. SwitchC needs to have the VTP domain name configured.

E. SwitchB is in transparent mode.

Answer: B

Explanation/Reference:

vtp同步需要在一個domain之內的交換局同步.並且password需要一直。B和D都可以理解，但是題庫要求要對狀態的描述，從邏輯上來說B正確

QUESTION 178

Refer to the exhibit. The network administrator has discorvered that the VLAN configuration of SwitchC is not synchronized with the rest of the switched network. Why is SwitchC not receiving VTP updates?(4.3.2-5)

A. SwitchB is not relaying VTP advertisements to SwitchC.

B. SwitchC has fewer existing VLANs than does SwitchA.

C. SwitchA supports a greater number of VLANs than does SwitchC.

D. SwitchC has revision number higher than that being advertised

E. SwitchC should be operating in VTP server mode to recevie VTP updates.

F. SwitchB should be operating in VTP server or client mode to relay VTP updates.

Answer: D

Explanation/Reference:

vtp在通告或者同步vlan資訊的時候是高的去通告其他所有的都同步高的,並不是同步server的

QUESTION 179

How many subnets can be gained by subnetting 172.17.32.0/23 into a /27 mask, and how many usable Host addresses will there be per subnet?(6.5.3-1)

A. 8 subnets, 31 hosts

B. 8 subnets, 32 hosts

C. 16 subnets, 30 hosts

D. 16 subnets, 32 hosts

E. A Class B address can't be subnetted into the fourth octet.

Answer: C

Explanation/Reference:

23-27為的遮罩.那麼將接用4位元主機位元所以可以劃分成16個子網.27為遮罩的可用地址為2的5次方-2=30個可用主機位址

QUESTION 180

A TCP/IP Transfer is diagrammed in the exhibit. A window size of three has been negotiated for this transfer. Which message will be returned from the receiver to the sender as part of this TCP/IP transfer?(4.2.3-1)

A. Send ACK 1-3

B. Send ACK 3

C. Send ACK 4

D. Send ACK 4-6

E. Send ACK 6

F. Send ACK 7

Answer: C

Explanation/Reference:

考點：考查TCP的確認機制。在接收到第n個資料包後，發送給對方的資料序列號為n+1，

一次性確認前面n個數據包。

綜上所述，發送ack=4的資料包。

QUESTION 181

Why is flash memory erased prior to upgrading the IOS image from the TFTP server?

A. The router cannot verify that the Cisco IOS image currently in flash is valid(11.2.3-1)

B. Flash memory on Cisco routers can contain only a single IOS image.

C. Erasing current flash content is requested during the copy dialog.

D. In order for the router to use the new image as the default, it must be the only IOS image in flash.

Answer: C

Explanation/Reference:

QUESTION 182

Refer to the exhibit. What is the meaning of the term dynamic as displayed in the output of the show frame-relay map command shown?(3.1.5-1)

A. The Serial0/0 interface is passing traffic.

B. The DLCI 100 was dynamically allocated by the router.

C. The Serial0/0 interface acquired by the IP address of 172.16.3.1 from a DHCP server.

D. The DLCI 100 will be dynamically changed as required to adapt to changes in the Frame Relay cloud.

E. The mapping between DLCI 100 and the end station IP address 172.16.3.1 was learned through inverse ARP.

Answer: E

Explanation/Reference:

dynamic意思是映射是通過翻轉ARP獲取，而不是手動設置

QUESTION 183

What can be done to secure the virtual terminal interfaces on a router? (Choose two.)(11.2.2-2)(5.1.3.1)

A. Administratively shut down the interface.

B. Physically secure the interface.

C. Create an access list and apply it to the virtual terminal interfaces with the access-group command.

D. Configure a virtual terminal password and login process.

E. Enter an access list and apply it to the virtual terminal interfaces using the access-class command.

Answer: DE

Explanation/Reference:

保障vty線路的安全,第一可以配置密碼.第二可以使用ACL來控制訪問的IP位址.但是ACL在vty線路下調用的命令是access-class,這個大家需要注意

QUESTION 184

Refer to the exhibit. In this VLSM addressing scheme, what summary address would be sent from router A?(6.5.3-1)

A. 172.16.0.0 /16

B. 172.16.0.0 /20

C. 172.16.0.0 /24

D. 172.32.0.0 /16

E. 172.32.0.0 /17

F. 172.64.0.0 /16

Answer: A

Explanation/Reference:

QUESTION 185

Refer to the exhibit. The show vtp status command is executed at a switch that is generating the exhibit output. Which statement is true for this switch?(4.2.4-1)

A. The switch forwards its VLAN database to other switches in the ICND VTP domain.

B. The configuration revision number increments each time the VLAN database is updated.

C. The switch forwards VTP updates that are sent by other switches in the ICND domain.

D. The VLAN database is updated when VTP information is received from other switches.

Answer: C

Explanation/Reference:

透明模式下revision number不增加，不接受更新但能轉發更新

QUESTION 186

Refer to the exhibit. Given the output for this command, if the router ID has not been manually set, what router ID will OSPF use for this RouterD?(11.2.4-3)(11.2.4-2)

A. 10.1.1.2

B. 10.154.154.1

C. 172.16.5.1

D. 192.168.5.3

Answer: C

Explanation/Referenc

正確答案為:C 選取最大LOOPBACK口地址

QUESTION 187

Refer to the exhibit. What could be possible causes for the "Serial0/0 is down" interface status?

(Choose two.)(8.2.2-1)(11.2.4-2)

A. A Layer 1 problem exists.

B. The bandwidth is set too low.

C. A protocol mismatch exists.

D. An incorrect cable is being

used.

E. There is an incorrect IP address on the Serial 0/0 interface.

Answer: AD

Explanation/Reference:

介面雙down，證明物理層就有問題，所以這一題主要排查的問題首先是物理層

QUESTION 188

Before installing a new, upgraded version of the IOS, what should be checked on the router, and which command should be used to gather this information? (Choose two.)(4.5.3-1)

A. the amount of available ROM

B. the amount of available flash and RAM memory

C. the version of the bootstrap software present on the router

D. show version

E. show processes

F. show running-config

Answer: BD

Explanation/Reference:

在升級IOS的時候，我們主要注意以下幾點：

1.flash空間是否充足

2.查看設備的版本對是否支援IOS

QUESTION 189

Refer to the exhibit. Which two statements are true about interVLAN routing in the topology that is shown in the exhibit? (Choose two.)(6.1.1-3)

A. Host E and host F use the same IP gateway address.

B. Router1 and Switch2 should be connected via a crossover cable.

C. Router1 will not play a role in communications between host A and host D.

D. The FastEthernet 0/0 interface on Router1 must be configured with subinterfaces.

E. Router1 needs more LAN interfaces to accommodate the VLANs that are shown in the exhibit.

F. The FastEthernet 0/0 interface on Router1 and Switch2 trunk ports must be configured using the same encapsulation type.

Answer: DF

Explanation/Reference:

在單臂路由的時候，首先路由器的介面要配置子介面針對每一個vlan一個邏輯介面，其次交換機的介面要配置trunk，並且封裝要和路由器介面的封裝一致

QUESTION 190

Refer to the exhibit. Which two statements are true about the loopback address that is configured on RouterB? (Choose two.)(11.2.4-2)(11.2.4-1)

A. It ensures that data will be forwarded by RouterB.

B. It provides stability for the OSPF process on RouterB.

C. It specifies that the router ID for RouterB should be 10.0.0.1.

D. It decreases the metric for routes that are advertised from RouterB.

E. It indicates that RouterB should be elected the DR for the LAN.

Answer: BC

Explanation/Reference:

loopback口作為在ospf當中，第一個ospf優先選擇loopback作為RID，loopback作為RID可以保障ospf的穩定性，因為loopback介面只有當路由器不可用的時候才會down