QUESTION 191

A network administrator is explaining VTP configuration to a new technician. What should the network administrator tell the new technician about VTP configuration? (Choose three.)(4.2.4-1)

A. A switch in the VTP client mode cannot update its local VLAN database.

B. A trunk link must be configured between the switches to forward VTP updates.

C. A switch in the VTP server mode can update a switch in the VTP transparent mode.

D. A switch in the VTP transparent mode will forward updates that it receives to other switches.

E. A switch in the VTP server mode only updates switches in the VTP client mode that have a higher VTP revision number.

F. A switch in the VTP server mode will update switches in the VTP client mode regardless of the configured VTP domain membership.

Answer: ABD

Explanation/Reference:

VTP的三個角色，以及VTP的特性，第一Client不能夠增刪改vlan資訊，也就是不能update自己本地的vlandatabase，第二在配置vtp的兩台交換機中間必須是trunk口用來傳遞vtp的通告以及vtp的根新第三在transparent模式的交換機講轉發它所收到的vtp更新以及vtp通告資訊

QUESTION 192

Which two locations can be configured as a source for the IOS image in the boot system command?(Choose two.)(2.3.8-2)(2.3.8-1)

A. RAM

B. NVRAM

C. flash memory

D. HTTP server

E. TFTP server

F. Telnet server

QUESTION 193

What are two reasons a network administrator would use CDP? (Choose two.)(2.3.3-1)

A. to verify the type of cable interconnecting two devices

B. to determine the status of network services on a remote device

C. to obtain VLAN information from directly connected switches

D. to verify Layer 2 connectivity between two devices when Layer 3 fails

E. to obtain the IP address of a connected device in order to telnet to the device

F. to determine the status of the routing protocols between directly connected routers

Answer: DE

Explanation/Reference:

CDP是一個二層協議，可以用來確認二層的連通性，以及查看遠端3層的IP地址

QUESTION 194

Refer to the exhibit. Both switches are using a default configuration. Which two destination addresses will host 4 use to send data to host 1? (Choose two.)(1.4.5-2)

A. the IP address of host 1

B. the IP address of host 4

C. the MAC address of host 1

D. the MAC address of host 4

E. the MAC address of the Fa0/0 interface of the R1 router

F. the MAC address of the Fa0/1 interface of the R1 router

Answer: AF

Explanation/Reference:

E太網在進行定址的時候，通過ARP請求道閘道的MAC位址，將其封裝到Frame的dest地址，Packet的source ip 和dest ip封裝的事自己的IP作為source 需要訪問的終點的IP作為dest IP

QUESTION 195

Refer to the exhibit. The router has been configured with these commands:

hostname Gateway

interface FastEthernet 0/0

ip address 198.133.219.14 255.255.255.248

no shutdown

interface FastEthernet 0/1

ip address 192.168.10.254 255.255.255.0

no shutdown

interface Serial 0/0

ip address 64.100.0.2 255.255.255.252

no shutdown

ip route 0.0.0.0 0.0.0.0 64.100.0.1

What are the two results of this configuration? (Choose two.)(7.2.2-1)

A. The default route should have a next hop address of 64.100.0.3.

B. Hosts on the LAN that is connected to FastEthernet 0/1 are using public IP addressing.

C. The address of the subnet segment with the WWW server will support seven more servers.

D. The addressing scheme allows users on the Internet to access the WWW server.

E. Hosts on the LAN that is connected to FastEthernet 0/1 will not be able to access the Internet without address translation.

Answer: DE

Explanation/Reference:

內網的私有網段要想訪問外部，必須使用NAT位址轉換。公網是不為私有網段路由的

QUESTION 196

A company is installing IP phones. The phones and office computers connect to the same device. To ensure maximum throughput for the phone data, the company needs to make sure that the phone traffic is on a different network from that of the office computer data traffic. What is the best network device to which to directly connect the phones and computers, and what technology should be implemented on this

device? (Choose two.)(3.1.3-1)

A. hub

B. router

C. switch

D. STP

E. subinterfaces

F. VLAN

Answer: CF

Explanation/Reference:

本題考查的事voice的基本性質，在vocie的時候我們需要一個switch並且配置voice vlan

QUESTION 197

What are two benefits of using VTP in a switching environment? (Choose two.)(4.1.1-3)

A. It allows switches to read frame tags.

B. It allows ports to be assigned to VLANs automatically.

C. It maintains VLAN consistency across a switched network.

D. It allows frames from multiple VLANs to use a single interface.

E. It allows VLAN information to be automatically propagated throughout the switching environment.

Answer: CE

Explanation/Reference:

VTP可以讓交換網路中位置vlan的一致性，也就是版本號的同步

QUESTION 198

Which two statements are true about the command ip route 172.16.3.0 255.255.255.0 192.168.2.4?(Choose two.)(2.4.1-2)(11.1.7-1)(3.4.3-1)

A. It establishes a static route to the 172.16.3.0 network.

B. It establishes a static route to the 192.168.2.0 network.

C. It configures the router to send any traffic for an unknown destination to the 172.16.3.0 network.

D. It configures the router to send any traffic for an unknown destination out the interface with the address

192.168.2.4.

E. It uses the default administrative distance.

F. It is a route that would be used last if other routes to the same destination exist.

Answer: AE

Explanation/Reference:

這條命令是靜態的為172.16.3.0建立一個路由，並且沒有指定它的AD，使用的事默認AD

QUESTION 199

What are two advantages of Layer 2 Ethernet switches over hubs? (Choose two.)(9.6.1-1)(9.6.2-2)

A. decreasing the number of collision domains

B. filtering frames based on MAC addresses

C. allowing simultaneous frame transmissions

D. increasing the size of broadcast domains

E. increasing the maximum length of UTP cabling between devices

Answer: BC

Explanation/Reference:

SW和HUB的區別在於，SW 可以對frame給予mac位址的過濾，而HUB只是傻瓜的轉發，沒有MAC地址學習的功能，SW是一個全雙工的設備。而hub工作在半雙工當中所以hub不支援同時傳輸Frame，它使用的是CSMA/CD機制

QUESTION 200

Refer to the exhibit. A network associate needs to configure the switches and router in the graphic so that the hosts in VLAN3 and VLAN4 can communicate with the enterprise server in VLAN2. Which two Ethernet segments would need to be configured as trunk links? (Choose two.)(6.1.1-2)

A. A

B. B

C. C

D. D

E. E

F. F

Answer: CF

Explanation/Reference:

本題考查的事vlan間路由的問題。要讓不同的vlan進行通信。那麼SW與SW之間的鏈路，SW與router之間的鏈路都需要配置成為trunk，用來傳遞不同vlan的資料

QUESTION 201

Which two values are used by Spanning Tree Protocol to elect a root bridge? (Choose two.)(5.2.3-1)

A. amount of RAM

B. bridge priority

C. IOS version

D. IP address

E. MAC address

F. speed of the links

Answer: BE

Explanation/Reference:

STP在選舉RB的時候依靠的事橋ID，橋ID包含兩個部分組成一個是橋優先順序另一部分就是橋mac

QUESTION 202

Refer to the exhibit. Assume that the routing protocol referenced in each choice below is configured with its default settings and the given routing protocol is running on all the routers. Which two conditional statements accurately state the path that will be chosen between networks 10.1.0.0 and 10.3.2.0 for the routing protocol mentioned? (Choose two.)(11.3.1-1)(4.1.1-1)(3.3.1-1)

A. If OSPF is the routing protocol, the path will be from R1 to R3 to R4 to R5.

B. If OSPF is the routing protocol, the path will be from R1 to R2 to R5.

C. If OSPF is the routing protocol, the path will be from R1 to R5.

D. If RIPv2 is the routing protocol, the path will be from R1 to R3 to R4 to R5.

E. If RIPv2 is the routing protocol, the path will be from R1 to R5.

Answer: AE

Explanation/Reference:

路由協議選路的問題。上圖表明瞭鏈路帶寬，如果是使用RIP協議的話。

那麼它選路和鏈路帶寬沒有關係。使用的是hop，所以選擇hop最小的，而ospf在進行選路過程中看的事帶寬，使用10的8次方處以帶寬計算出來的度量最小的

QUESTION 203

Refer to the exhibit. A network administrator is adding two new hosts to SwitchA.

Which three values could be used for the configuration of these hosts? (Choose three.)(6.2.2-1)(6.5.1-1)

A. host A IP address: 192.168.1.79

B. host A IP address: 192.168.1.64

C. host A default gateway: 192.168.1.78

D. host B IP address: 192.168.1.128

E. host B default gateway: 192.168.1.129

F. host B IP address: 192.168.1.190

Answer: ACF

Explanation/Reference:

單臂路由的問題。單臂路由子介面的IP就是主機的default-gateway

QUESTION 204

A network administrator changes the configuration register to 0x2142 and reboots the router. What are two results of making this change? (Choose two.)(2.4.1-5)

A. The IOS image will be ignored.

B. The router will prompt to enter initial configuration mode.

C. The router will boot to ROM.

D. Any configuration entries in NVRAM will be ignored.

E. The configuration in flash memory will be booted.

Answer: BD

Explanation/Reference:

0x2142寄存器值，是在啟動的時候不調用nvram中的start-config，會按照初始狀態進行啟動

QUESTION 205

Refer to the exhibit. The networks connected to router R2 have been summarized as a 192.168.176.0/21 route and sent to R1. Which two packet destination addresses will R1 forward to R2? (Choose two.)(6.5.1-1)

A. 192.168.194.160

B. 192.168.183.41

C. 192.168.159.2

D. 192.168.183.255

E. 192.168.179.4

F. 192.168.184.45

Answer: BE

Explanation/Reference:

子網問題，192.168.176.0/21的範圍是：192.168.176.0-192.168.183.255

QUESTION 206

Which three statements are typical characteristics of VLAN arrangements? (Choose three.)(6.1.1-1)

A. A new switch has no VLANs configured.

B. Connectivity between VLANs requires a Layer 3 device.

C. VLANs typically decrease the number of collision domains.

D. Each VLAN uses a separate address space.

E. A switch maintains a separate bridging table for each VLAN.

F. VLANs cannot span multiple switches.

Answer: BDE

Explanation/Reference:

vlan的特徵：

vlan之間默認是不能夠通信的，需喲啊通信則需要第三層設備進行路由

一個vlan就是一個邏輯子網sw為每一個vlan維護一個單獨的橋接表，這個其實比較難以理解，但是我們知道一個vlan就是一個邏輯子網，那麼不同的vlan對於交換機而言當然是不同的表項了。

QUESTION 207

QoS policies are applied on the switches of a LAN. Which type of command will show the effects of the policy in real time?(4.5.6-4)

A. show command

B. debug command

C. configuration command

D. rommon command

Answer: B

Explanation/Reference:

問什麼命令能夠即時的查看policies的結果。即時查看我們只能開啟debug來看是否有匹配到Qos的policies

QUESTION 208

Which command will show the MAC addresses of stations connected to switch ports?(9.7.1-1)(9.7.2-1)

A. show mac-address

B. show arp

C. show table

D. show switchport

Answer: B

Explanation/Reference:

查看station連接switch的埠。我們只需要查看arp的解析就OK了。也就是show arp，因為stations有流量需要訪問。就會向switch發送arp的請求，show mac address-table也是可以看得到的。但是A的命令是錯誤的

QUESTION 209

What is the name of the VTP mode of operation that enables a switch to forward only VTP advertisements while still permitting the editing of local VLAN information?(4.2.4-1)

A. server

B. client

C. tunnel

D. transparent

Answer: D

Explanation/Reference:

哪一種vtp的mode轉發vtp通告並且維護自己本地的vlan資訊，這種情況就是透明模式的特點

QUESTION 210

Which port state is introduced by Rapid-PVST?(5.4.7-1)

A. learning

B. listening

C. discarding

D. forwarding

Answer: C

Explanation/Reference:

Rpvst新增了一種埠角色----discarding

QUESTION 211

What speeds must be disabled in a mixed 802.11b/g WLAN to allow only 802.11g clients to connect?(7.1.2-1)

A. 6, 9, 12, 18

B. 1, 2, 5.5, 6

C. 5.5, 6, 9, 11

D. 1, 2, 5.5, 11

Answer: D

Explanation/Reference:

1,2,5.5,11是802.11b的傳輸速率

QUESTION 212

Refer to the exhibit. Which VLSM mask will allow for the appropriate number of host addresses for Network A?(6.5.3-5)

A. /25

B. /26

C. /27

D. /28

Answer: A

Explanation/Reference:

66個主機的話。那麼我們2的X次方-2要大於等於66，很容易算出遮罩為/25

QUESTION 213

Refer to the exhibit. Which subnet mask will place all hosts on Network B in the same subnet with the least amount of wasted addresses?(6.5.1-1)

A. 255.255.255.0

B. 255.255.254.0

C. 255.255.252.0

D. 255.255.248.0

Answer: B

Explanation/Reference:

310個主機,那麼2的X次方-2>=310

QUESTION 214

Refer to the exhibit. Which mask is correct to use for the WAN link between the routers that will provide connectivity while wasting the least amount of addresses?(6.5.1-2)(6.5.3-5)

A. /23

B. /24

C. /25

D. /30

Answer: D

Explanation/Reference:

中間的point-to-point鏈路最節省地址的遮罩就是/30

QUESTION 215

Refer to the exhibit. What is the most appropriate summarization for these routes?(2.6.1-1)

A. 10.0.0.0 /21

B. 10.0.0.0 /22

C. 10.0.0.0 /23

D. 10.0.0.0 /24

Answer: B

Explanation/Reference:

做匯總.顯然我們要在第三個位組做匯總,通過二進位換算我們可以知道0-3最多只需要2bit就能表示0-3所有的數字.所以24-2得到遮罩/22

QUESTION 216

Which two tasks does the Dynamic Host Configuration Protocol perform? (Choose two.)(7.1.2-1)

A. Set the IP gateway to be used by the network.

B. Perform host discovery used DHCPDISCOVER message.

C. Configure IP address parameters from DHCP server to a host.

D. Provide an easy management of layer 3 devices.

E. Monitor IP performance using the DHCP server.

F. Assign and renew IP address from the default pool.

Answer: CF

Explanation/Reference:

考查的是DHCP提供的服務.DHCP主要的服務時向主機提供ip位址,並且如果你要添加或者更新ip 位址.需要在dhcp pool中添加就OK了

QUESTION 217

Which two benefits are provided by using a hierarchical addressing network addressing scheme?(1.1.1-3)

(Choose two.)

A. reduces routing table entries

B. auto-negotiation of media rates

C. efficient utilization of MAC addresses

D. dedicated communications between devices

E. ease of management and troubleshooting

Answer: AE

Explanation/Reference:

提供分層的網路位址規劃,第一個我們容易即時路由匯總從而減少路由條目,第二給我們在管理方面還錯誤排查方面提供了便利

QUESTION 218

Which two benefits are provided by creating VLANs? (Choose two.)(6.1.1-1)(6.1.1-2)

A. added security

B. dedicated bandwidth

C. provides segmentation

D. allows switches to route traffic between subinterfaces

E. contains collisions

Answer: AC

Explanation/Reference:

vlan的主要特點第一個提供網路安全因為默認vlan間是不能夠通信的,第二個提供了網路分段.因為一個vlan就是一個邏輯子網

QUESTION 219

Which two link protocols are used to carry multiple VLANs over a single link? (Choose two.)(3.2.3-1)

A. VTP

B. 802.1q

C. IGP

D. ISL

E. 802.3u

Answer: BD

Explanation/Reference:

trunk的封裝就兩個,一個位ieee公有的802.1q一個位cisco私有的isl封裝類型

QUESTION 220

Which two protocols are used by bridges and/or switches to prevent loops in a layer 2 network? (Choose two.)(5.1.2-1)(5.4.3-1)

A. 802.1d

B. VTP

C. 802.1q

D. STP

E. SAP

Answer: AD

Explanation/Reference:

阻止二層的環路就是生成樹協議.這裏802.1d以及stp都是阻止二層環路的有效手段

QUESTION 221

On the network 131.1.123.0/27, what is the last IP address that can be assigned to a host?(6.5.1-2)

A. 131.1.123.30

B. 131.1.123.31

C. 131.1.123.32

D. 131.1.123.33

Answer: A

Explanation/Reference:

131.1.123.0/27的塊大小為32有效位址範圍為131.1.123.1-131.1.123.30

QUESTION 222

The ip subnet zero command is not configured on a router. What would be the IP address of Ethernet0/0 using the first available address from the sixth subnet of the network 192.168.8.0/29?(6.5.1-2)

A. 192.168.8.25

B. 192.168.8.41

C. 192.168.8.49

D. 192.168.8.113

Answer: C

Explanation/Reference:

192.168.8.0/29的塊大小為8那麼第六個subnet則為6*8=48第一個可用地址則為192.168.8.49

QUESTION 223

For the network 192.0.2.0/23, which option is a valid IP address that can be assigned to a host?(6.5.1-1)

A. 192.0.2.0

B. 192.0.2.255

C. 192.0.3.255

D. 192.0.4.0

Answer: B

Explanation/Reference:

192.0.2.0/23的塊大小為512可用地址範圍為192.0.2.1-192.0.3.254

QUESTION 224

How many addresses for hosts will the network 124.12.4.0/22 provide?(6.5.1-1)

A. 510

B. 1022

C. 1024

D. 2048

Answer: B

Explanation/Reference:

124.12.4.0/22的塊大小為2的10次方=1024 可用地址則為1024-2=1022

QUESTION 225

Where does routing occur within the DoD TCP/IP reference model?(1.4.2-1)(1.4.2-2)

A. application

B. internet

C. network

D. transport

Answer: B

Explanation/Reference:

路由在tcp/ip協議棧的internet層

注意是DOD模型不是OSI模型

QUESTION 226

Which VTP mode is capable of creating only local VLANs and does not synchronize with other switches inthe VTP domain?(4.2.3-4)(4.2.4-1)

A. client

B. dynamic

C. server

D. static

E. transparent

Answer: E

Explanation/Reference:

vtp transparent的功能為,不同步vlan資訊,轉發vtp通告可以在本地增刪改vlan資訊,維護本地的資料庫

QUESTION 227

Which switch would STP choose to become the root bridge in the selection process?(5.3.3-2)

A. 32768: 11-22-33-44-55-66

B. 32768: 22-33-44-55-66-77

C. 32769: 11-22-33-44-55-65

D. 32769: 22-33-44-55-66-78

Answer: A

Explanation/Reference:

stp選舉跟橋首先比較優先順序大小.然後比較mac位址大小

QUESTION 228

A switch is configured with all ports assigned to vlan 2 with full duplex FastEthernet to segment existing departmental traffic. What is the effect of adding switch ports to a new VLAN on the switch?(3.1.1-3)

A. More collision domains will be created.

B. IP address utilization will be more efficient.

C. More bandwidth will be required than was needed previously.

D. An additional broadcast domain will be created.

Answer: D

Explanation/Reference:

vlan的特性是對廣播域的分割.增加廣播域的數量,減少廣播域的大小因為一個vlan就是一個廣播域

QUESTION 229

Which two statements about the use of VLANs to segment a network are true? (Choose two.)(3.1.1-2)

A. VLANs increase the size of collision domains.

B. VLANs allow logical grouping of users by function.

C. VLANs simplify switch administration.

D. VLANs enhance network security.

Answer: BD

Explanation/Reference:

vlan是一個邏輯分組,

A vlan =一個邏輯子網=一個廣播域

並且vlan間默認是不通信的,從而間接保證了網路的安全

QUESTION 230

On corporate network, hosts on the same VLAN can communicate with each other, but theyare unable to communicate with hosts on different VLANs. What is needed to allow communication between the VLANs?(6.1.2-1)

A. a router with subinterfaces configured on the physical interface that is connected to the switch

B. a router with an IP address on the physical interface connected to the switch

C. a switch with an access link that is configured between the switches

D. a switch with a trunk link that is configured between the switches

Answer: A

Explanation/Reference:

考查單臂路由的基本配置,在配置單臂路由的時候.我們要針對每一個需要路由的vlan建立一個子介面.並且配上IP地址.此ip即為host的閘道

QUESTION 231

When a DHCP server is configured, which two IP addresses should never be assignable to hosts? (Choose two.)(6.2.6-1)

A. network or subnetwork IP address

B. broadcast address on the network

C. IP address leased to the LAN

D. IP address used by the interfaces

E. manually assigned address to the clients

F. designated IP address to the DHCP server

Answer: AB

Explanation/Reference:

不能夠被主機使用的ip也就是兩個.一個網路號或者子網號,還一個就是廣播位址

QUESTION 232

Which statement describes the process of dynamically assigning IP addresses by the DHCP server?(7.1.2-1)

A. Addresses are allocated after a negotiation between the server and the host to determine the length of the agreement.

B. Addresses are permanently assigned so that the hosts uses the same address at all times.

C. Addresses are assigned for a fixed period of time, at the end of the period, a new request for an address must be made.

D. Addresses are leased to hosts, which periodically contact the DHCP server to renew the lease.

Answer: D

Explanation/Reference:

DHCP的主要作用就是向主機提供位址租約,並且租約如果快到期了.主機需要向DHCPserver重新續租

QUESTION 233

Which network protocol does DNS use?(3.3.1-1)

A. FTP

B. TFTP

C. TCP

D. UDP

E. SCP

Answer: D

Explanation/Reference:

此題答案並不嚴格,嚴格講DNS有兩個埠號一個為TCP一個為UDP並且埠號都為53DNS解析用UDP 效率高但不能保證完整性DNS伺服器之間同步使用TCP保證資料正確

QUESTION 234

When two hosts are trying to communicate across a network, how does the host originating the communication determine the hardware address of the host that it wants to "talk" to?(7.7.1-1)

A. RARP request

B. Show Network Address request

C. Proxy ARP request

D. ARP request

E. Show Hardware Address request

Answer: D

Explanation/Reference:

host在建立通信,需要通過arp請求來請求道目標IP的mac位址.

QUESTION 235

Host 1 is trying to communicate with Host 2. The e0 interface on Router C is down. Which of the following are true? (Choose two.)(6.6.1-1)(11.3.2-1)(11.3.4-1)

A. Router C will use ICMP to inform Host 1 that Host 2 cannot be reached.

B. Router C will use ICMP to inform Router B that Host 2 cannot be reached.

C. Router C will use ICMP to inform Host 1, Router A, and Router B that Host 2 cannot be reached.

D. Router C will send a Destination Unreachable message type.

E. Router C will send a Router Selection message type.

F. Router C will send a Source Quench message type.

Answer: AD

Explanation/Reference:

因為RC的e0口down，所以主機1的資料包到達RC後，RC向主機返回一個icmp的目的地不可達資訊

QUESTION 236

To configure the VLAN trunking protocol to communicate VLAN information between two switches, whattwo requirements must be met? (Choose two.)(4.2.1-1)(4.2.2-2)(4.2.4-1)

A. Each end of the trunk line must be set to IEEE 802.1E encapsulation.

B. The VTP management domain name of both switches must be set the same.

C. All ports on both the switches must be set as access ports.

D. One of the two switches must be configured as a VTP server.

E. A rollover cable is required to connect the two switches together.

F. A router must be used to forward VTP traffic between VLANs.

Answer: BD

Explanation/Reference:

兩個交換機之間使用VTP的前提？

A：必須使用802.1e封裝，錯誤。

B：VTP功能變數名稱必須一致，正確。

C：所有埠必須設置為access口，錯誤。

D：其中一個必須設置為sever模式。正確

E：使用反轉先連接，錯誤。

F：必須使用一個路由中轉資料，錯誤。

QUESTION 237

Which of the following describe the process identifier that is used to run OSPF on a router? (Choose two.)(11.2.3-1)

A. It is locally significant.

B. It is globally significant.

C. It is needed to identify a unique instance of an OSPF database.

D. It is an optional parameter required only if multiple OSPF processes are running on the router.

E. All routers in the same OSPF area must have the same process ID if they are to exchange routing information.

Answer: AC

Explanation/Reference:

配置OSPF的時候，ospf後面的進程號的意義：rouer ospf #

A：本地有意義，正確

B：全局有意義，錯誤

C：用於確定一個ospf資料庫實例，正確。本地路由器上一個進程號一個資料庫。

D：這是一個可選參數，錯誤

E：區域中的所有路由必須使用相同的進程號，錯誤。

QUESTION 238

What functions do routers perform in a network? (Choose two.)(1.1.1-3)

A. packet switching

B. access layer security

C. path selection

D. VLAN membership assignment

E. bridging between LAN segments

F. microsegmentation of broadcast domains

Answer: AC

Explanation/Reference:

路由器的作用

A：轉發資料，正確

B：接入層安全，錯誤。通常由交換機實現。

C：路徑選擇，正確

D：VLAN成員分配，錯誤。

E：橋接不同網段，錯誤。不夠準確

F：縮小廣播域的分割，錯誤。

QUESTION 239

Which series of commands will configure router R1 for LAN-to-LAN communication with router R2? The enterprise network address is 192.1.1.0/24 and the routing protocol in use is RIP. (Choose three.)(11.2.4-2)(6.5.1-2)(5.2.3-1)(5.2.3-2)

A. R1(config)# interface ethernet 0

R1(config-if)# ip address 192.1.1.129 255.255.255.192

R1(config-if)# no shutdown

B. R1(config)# interface ethernet 0

R1(config-if)# ip address 192.1.1.97 255.255.255.192

R1(config-if)# no shutdown

C. R1(config)# interface serial 0

R1(config-if)# ip address 192.1.1.4 255.255.255.252

R1(config-if)# clock rate 56000

D. R1(config)# interface serial 0

R1(config-if)# ip address 192.1.1.6 255.255.255.252

R1(config-if)# no shutdown

E. R1(config)# router rip

R1(config-router)# network 192.1.1.4

R1(config-router)# network 192.1.1.128

F. R1(config)# router rip

R1(config-router)# version 2

R1(config-router)# network 192.1.1.0

Answer: ADF

Explanation/Reference

QUESTION 240

A network administrator wants to ensure that only the server can connect to port Fa0/1 on a Catalyst switch. The server is plugged into the switch Fa0/1 port and the network administrator is about to bring the server online. What can the administrator do to ensure that only the MAC address of the server is allowed by switch port Fa0/1? (Choose two.)(2.4.6-1)(2.4.6-2)

A. Configure port Fa0/1 to accept connections only from the static IP address of the server.

B. Employ a proprietary connector type on Fa0/1 that is incompatible with other host connectors.

C. Configure the MAC address of the server as a static entry associated with port Fa0/1.

D. Bind the IP address of the server to its MAC address on the switch to prevent other hosts from spoofing the server IP address.

E. Configure port security on Fa0/1 to reject traffic with a source MAC address other than that of the server.

F. Configure an access list on the switch to deny server traffic from entering any port other than Fa0/1.

Answer: CE

Explanation/Reference:

伺服器接到f0/1口，有什麼方法使僅伺服器才能接入？

B：靜態配置ARP

E：port security

QUESTION 241

The internetwork infrastructure of company XYZ consists of a single OSPF area as shown in the graphic. There is concern that a lack of router resources is impeding internetwork performance. As part of examining the router resources, the OSPF DRs need to be known. All the router OSPF priorities are at the default and the router IDs are shown with each router. Which routers are likely to have been elected as

DR? (Choose two.) (11.4.2-2)

A. Corp-1

B. Corp-2

C. Corp-3

D. Corp-4

E. Branch-1

F. Branch-2

Answer: DF

Explanation/Reference:

DR的選舉

1、一個多路訪問網段中選舉

2、優先順序大的當選；優先順序相同，RID大的當選。

如圖中兩個橢圓的多路網路中選舉。

QUESTION 242

At which layers of the OSI model do WANs operate? (Choose two.)(1.2.1-1)

A. application layer

B. session layer

C. transport layer

D. network layer

E. datalink layer

F. physical layer

Answer: EF

Explanation/Reference:

QUESTION 243

Which routing protocols can be used within the enterprise network shown in the diagram? (Choose three.)(3.2.4-1)(3.2.2-1)

A. RIP v1

B. RIP v2

C. IGRP

D. OSPF

E. BGP

F. EIGRP

Answer: BDF

Explanation/Reference:

由圖中可知，使用IGP路由協定，需支援無類路由。

QUESTION 244

Refer to the graphic. R1 is unable to establish an OSPF neighbor relationship with R3. What are possible reasons for this problem? (Choose two.)(11.2.5-1)

A. All of the routers need to be configured for backbone Area 1.

B. R1 and R2 are the DR and BDR, so OSPF will not establish neighbor adjacency with R3.

C. A static route has been configured from R1 to R3 and prevents the neighbor adjacency from being established.

D. The hello and dead interval timers are not set to the same values on R1 and R3.

E. EIGRP is also configured on these routers with a lower administrative distance.

F. R1 and R3 are configured in different areas.

Answer: DF

Explanation/Reference:

影響OSPF鄰居形成的幾個因素：區域ID，hello時間，網段，認證等

A：所有的路由器需配置在區域1,錯誤

B：R1/R2是DR/BDR，所以OSPF不會和R3建立鄰居關係.

C：配置了靜態路由保護了鄰居的建立，錯誤。

D：hello包週期不一致，正確

E：配置了EIGRP。錯誤

F：R1和R3配置了不同的區域。正確。

QUESTION 245

Which statements are true about EIGRP successor routes? (Choose two.)(9.4.2-1)(9.4.3-1)

A. A successor route is used by EIGRP to forward traffic to a destination.

B. Successor routes are saved in the topology table to be used if the primary route fails.

C. Successor routes are flagged as "active" in the routing table.

D. A successor route may be backed up by a feasible successor route.

E. Successor routes are stored in the neighbor table following the discovery process.

Answer: AD

Explanation/Reference:

successor router是最優路徑的下一跳。

A:被用作轉發流量，正確。

B：保存在拓撲庫中，當主路由失敗時被使用，錯誤。

C：在路由表中顯示為active，錯誤。在資料庫中的顯示。

D：FS做為一個備份路由。正確

E：後繼路由器存儲在鄰居表中，錯誤。在拓撲庫中。

QUESTION 246

Refer to the diagram. All hosts have connectivity with one another. Which statements describe the addressing scheme that is in use in the network? (Choose three.)(6.1.1-3)(6.5.1-1)

A. The subnet mask in use is 255.255.255.192.

B. The subnet mask in use is 255.255.255.128.

C. The IP address 172.16.1.25 can be assigned to hosts in VLAN1

D. The IP address 172.16.1.205 can be assigned to hosts in VLAN1

E. The LAN interface of the router is configured with one IP address.

F. The LAN interface of the router is configured with multiple IP addresses.

Answer: BCF

Explanation/Reference:

1、子網路遮罩要使兩個IP處於不同網段。

255.255.255.128把一個C類劃分成兩個子網。

2、在路由器上配置子介面，用單臂路由實現通信。

QUESTION 247

Refer to the diagram. Which three statements describe the router port configuration and the switch port configuration as shown in the topology? (Choose three.)(6.1.2-1)(6.1.2-3)(3.3.2-2)

A. The Router1 WAN port is configured as a trunking port.

B. The Router1 port connected to Switch1 is configured using subinterfaces.

C. The Router1 port connected to Switch1 is configured as 10 Mbps.

D. The Switch1 port connected to Router1 is configured as a trunking port.

E. The Switch1 port connected to Host B is configured as an access port.

F. The Switch1 port connected to Hub1 is configured as full duplex.

Answer: BDE

Explanation/Reference:

1、使用單臂路由。

2、交換機互聯路由器的介面需配置成trunk模式.

3、交換機連主機的介面配置成access模式。

QUESTION 248

Which routing protocols will support the following IP addressing scheme? (Choose three.)(3.2.4-1)

Network 1 - 192.168.10.0 /26

Network 2 - 192.168.10.64 /27

Network 3 - 192.168.10.96 /27

Network 4 - 192.168.10.128 /30

Network 5 - 192.168.10.132 /30

A. RIP version 1

B. RIP version 2

C. IGRP

D. EIGRP

E. OSPF

Answer: BDE

Explanation/Reference:

必須使用支援無類路由的路由協定。

RIPv1,IGRP屬於有類路由協議。

QUESTION 249

Refer to the partial command output shown. Which two statements are correct regarding the router hardware? (Choose two.)(1.1.4-2)

A. Total RAM size is 32 KB.

B. Total RAM size is 16384 KB (16 MB).

C. Total RAM size is 65536 KB (64 MB).

D. Flash size is 32 KB.

E. Flash size is 16384 KB (16 MB).

F. Flash size is 65536 KB (64 MB).

Answer: CE

Explanation/Reference:

QUESTION 250

The Ethernet networks connected to router R1 in the graphic have been summarized for router R2 as192.1.144.0/20. Which of the following packet destination addresses will R2 forward to R1, according to this summary? (Choose two.)(6.5.1-2)

A. 192.1.159.2

B. 192.1.160.11

C. 192.1.138.41

D. 192.1.151.254

E. 192.1.143.145

F. 192.1.1.144

Answer: AD

Explanation/Reference:

192.1.144.0/20所屬網路的範圍是：192.1.144.0/20-192.1.159.255/20