QUESTION 251

The show interfaces serial 0/0 command resulted in the output shown in the graphic. What are possible causes for this interface status? (Choose three.)(8.4.3-3)

A. The interface is shut down.

B. No keepalive messages are received.

C. The clockrate is not set.

D. No loopback address is set.

E. No cable is attached to the interface.

F. There is a mismatch in the encapsulation type.

Answer: BCF

Explanation/Reference:

串口通過keepalive包保活埠。協定層down通常是時鐘，封裝，線纜使用不對造成。

QUESTION 291

Which two of these statements are true of IPv6 address representation? (Choose two.)(7.3.2-2)

A. There are four types of IPv6 addresses: unicast, multicast, anycast, and broadcast.

B. A single interface may be assigned multiple IPv6 addresses of any type.

C. Every IPv6 interface contains at least one loopback address.

D. The first 64 bits represent the dynamically created interface ID.

E. Leading zeros in an IPv6 16 bit hexadecimal field are mandatory.

Answer: BC

Explanation/Reference:

每個Ipv6介面至少包括一個環回口位址：由mac位址轉換不用手動設置

QUESTION 292

What are three basic parameters to configure on a wireless access point? (Choose three.)(7.3.1-2)(7.3.1-3)

A. SSID

B. RTS/CTS

C. AES-CCMP

D. TKIP/MIC

E. RF channel

F. authentication method

Answer: AEF

Explanation/Reference:

wireless三要素 ssid

RF channel 以及認證需求

QUESTION 252

Which of the following describe private IP addresses? (Choose two.)(6.2.5-1)

A. addresses chosen by a company to communicate with the Internet

B. addresses that cannot be routed through the public Internet

C. addresses that can be routed through the public Internet

D. a scheme to conserve public addresses

E. addresses licensed to enterprises or ISPs by an Internet registry organization

Answer: BD

Explanation/Reference:

私有地址的特性

A：可以和互聯網通信，錯誤

B：不能和互聯網通信，正確。

C：能被互聯網路由,錯誤

D：用於節省IP位址，正確。通過nat實現。

E：需要經過授權使用，錯誤

QUESTION 253

Refer to the exhibit. The network shown in the diagram is experiencing connectivity problems. Which of the following will correct the problems? (Choose two.)(6.3.3-1)

A. Configure the gateway on Host A as 10.1.1.1.

B. Configure the gateway on Host B as 10.1.2.254.

C. Configure the IP address of Host A as 10.1.2.2.

D. Configure the IP address of Host B as 10.1.2.2.

E. Configure the masks on both hosts to be 255.255.255.224.

F. Configure the masks on both hosts to be 255.255.255.240.

Answer: BD

Explanation/Reference:

考查VLSM。閘道和主機IP需在同一個子網。路由器子介面IP為這個vlan中主機的閘道

QUESTION 254

A network associate is trying to understand the operation of the FLD Corporation by studying the network in the exhibit. The associate knows that the server in VLAN 4 provides the necessary resources to support the user hosts in the other VLANs. The associate needs to determine which interfaces are access ports. Which interfaces are access ports? (Choose three.)(3.3.2-2)

A. Switch1 - Fa 0/2

B. Switch1 - Fa 0/9

C. Switch2 - Fa 0/3

D. Switch2 - Fa 0/4

E. Switch2 - Fa 0/8

F. Router - Fa 1/0

Answer: ACD

Explanation/Reference:

接主機的都需要設置為access口

QUESTION 255

Refer to the exhibit. Which destination addresses will Host A use to send data to Host B? (Choose two.)(1.4.5-2)

A. the IP address of Switch1

B. the IP address of Router1 Fa0/0

C. the IP address of HostB

D. the MAC address of Switch1

E. the MAC address of Router1 Fa0/0

F. the MAC address of HostB

Answer: CE

Explanation/Reference:

QUESTION 256

The network security policy requires that only one host be permitted to attach dynamically to each switch interface. If that policy is violated, the interface should shut down. Which two commands must the network administrator configure on the 2950 Catalyst switch to meet this policy? (Choose two.)(2.4.6-2)(2.4.6-1)

A. Switch1(config-if)# switchport port-security maximum 1

B. Switch1(config)# mac-address-table secure

C. Switch1(config)# access-list 10 permit ip host

D. Switch1(config-if)# switchport port-security violation shutdown

E. Switch1(config-if)# ip access-group 10

Answer: AD

Explanation/Reference:

QUESTION 257

Refer to the graphic. A static route to the 10.5.6.0/24 network is to be configured on the HFD router. Which commands will accomplish this? (Choose two.)(2.5.1-1)

A. HFD(config)# ip route 10.5.6.0 0.0.0.255 fa0/0

B. HFD(config)# ip route 10.5.6.0 0.0.0.255 10.5.4.6

C. HFD(config)# ip route 10.5.6.0 255.255.255.0 fa0/0

D. HFD(config)# ip route 10.5.6.0 255.255.255.0 10.5.4.6

E. HFD(config)# ip route 10.5.4.6 0.0.0.255 10.5.6.0

F. HFD(config)# ip route 10.5.4.6 255.255.255.0 10.5.6.0

Answer: CD

Explanation/Reference:

考點：靜態路由的配置方式

下一跳可以寫出口和鄰居IP

QUESTION 258

What should be done prior to backing up an IOS image to a TFTP server? (Choose three.)(4.5.3-1)(4.5.4-1)

A. Make sure that the server can be reached across the network.

B. Check that authentication for TFTP access to the server is set.

C. Assure that the network server has adequate space for the IOS image.

D. Verify file naming and path requirements.

E. Make sure that the server can store binary files.

F. Adjust the TCP window size to speed up the transfer.

Answer: ACD

Explanation/Reference:

考點：考查升級IOS時的注意事項

1、確保網路連通性，2、確保設備有存放IOS的空間，3、確認檔的路徑和名字

綜上所述，ACD。

QUESTION 259

What are the possible trunking modes for a switch port? (Choose three.)(3.2.3-1)

A. transparent

B. auto

C. on

D. desirable

E. client

F. forwarding

Answer: BCD

Explanation/Reference:

考點：考查交換機在trunk模式下的模式

auto：不會主動發DTP資訊

on：強制成為trunk，也主動發DTP資訊

desirable：DTP主動模式，發DTP和對方協商

QUESTION 260

The network 172.25.0.0 has been divided into eight equal subnets. Which of the following IP addresses Can be assigned to hosts in the third subnet if the ip subnet-zero command is configured on the router?(Choose three.)(6.5.1-3)

A. 172.25.78.243

B. 172.25.98.16

C. 172.25.72.0

D. 172.25.94.255

E. 172.25.96.17

F. 172.25.100.16

Answer: ACD

Explanation/Reference:

考點：子網劃分。172.25.0.0被劃分為8個子網，並且設備配置了ip subnet-zero這條命令，那個ip屬於第三個子網。劃分為8個子網，網路位元向主機位元借了三位元。八個子網分別為：

1、172.25.0.0/19

2、172.25.32.0/19

3、172.25.64.0/19

4、172.25.96.0/19

5、172.25.128.0/19

6、172.25.160.0/19

7、172.25.192.0/19

8、172.25.224.0/19

第三個子網的範圍為172.25.64.0/19-172.25.95.255/19

QUESTION 261

Refer to the graphic. Host 1 cannot receive packets from Host 2. Assuming that RIP v1 is the routing protocol in use, what is wrong with the IP configuration information shown? (Choose two.)(6.5.1-2)

A. The fa0/1 interface of router R2 has been assigned a broadcast address.

B. The fa0/1 network on router R2 overlaps with the LAN attached to R1.

C. Host 2 has been assigned the incorrect subnet mask.

D. Host 1 has been configured with the 255.255.248.0 subnet mask.

E. Host 2 on router R2 is on a different subnet than its gateway.

Answer: BC

Explanation/Reference:

A：f0/1被配置為廣播位址，錯誤。10.1.40.255/22的廣播地址是10.1.43.255/22

B：10.1.32.48/20的範圍是：10.1.32.0-10.1.47.255/0，和R2 f0/1的網段重合，正確。

C：正確

D：明顯和給出資訊不同

E：處於同一個網段，錯誤。

QUESTION 262

What are three valid reasons to assign ports to VLANs on a switch? (Choose three.)(3.1.1-3)

A. to make VTP easier to implement

B. to isolate broadcast traffic

C. to increase the size of the collision domain

D. to allow more devices to connect to the network

E. to logically group hosts according to function

F. to increase network security

Answer: BEF

Explanation/Reference:

考點：考查vlan的特性，選出三個把埠分配到vlan的原因。

A:使VTP更容易部署，錯誤，答非所問

B：限制廣播流量，正確

C：較少衝突域的範圍，錯誤，沒有這個作用

D：允許更多的設備接入，錯誤，答非所問

E：邏輯劃分網段，正確，一個vlan一個網段。

F：增加網路安全，正確。不同vlan不能直接通信。

QUESTION 263

Refer to the topology shown in the exhibit. Which ports will be STP designated ports if all the links are operating at the same bandwidth? (Choose three.)(5.2.3-1)

A. Switch A - Fa0/0

B. Switch A - Fa0/1

C. Switch B - Fa0/0

D. Switch B - Fa0/1

E. Switch C - Fa0/0

F. Switch C - Fa0/1

Answer: BCD

Explanation/Reference:

B的mac最小為根橋，B的倆埠為指定埠. A的F0/0和C的F0/0為根埠。A的mac較小，A的F0/1為指定埠，C的F0/1堵塞

QUESTION 264

Which statements describe two of the benefits of VLAN Trunking Protocol? (Choose two.)(4.2.1-1)

A. VTP allows routing between VLANs.

B. VTP allows a single switch port to carry information to more than one VLAN.

C. VTP allows physically redundant links while preventing switching loops.

D. VTP simplifies switch administration by allowing switches to automatically share VLAN configuration information.

E. VTP helps to limit configuration errors by keeping VLAN naming consistent across the VTP domain.

F. VTP enhances security by preventing unauthorized hosts from connecting to the VTP domain.

Answer: DE

Explanation/Reference:

考點：考查VTP的特性

A:允許vlan間路由，錯誤，答非所問

B：允許單一鏈路承載多個vlan資訊，錯誤，答非所問

C：允許物理冗餘，錯誤，這個是STP的功能。

D：自動分享vlan配置資訊，正確。

E：減少vtp域中vlan名字錯誤配置，正確，僅需配置一個，其他的自帶學習，減少配置量從而減少錯誤概率。

F：增強安全性，未通過認證的主機不能加入。錯誤。沒有這個功能。

QUESTION 265

What are two results of entering the Switch(config)# vtp mode client command on a Catalyst switch? (Choose two.)(4.2.4-1)

A. The switch will ignore VTP summary advertisements.

B. The switch will forward VTP summary advertisements.

C. The switch will process VTP summary advertisements.

D. The switch will originate VTP summary advertisements.

E. The switch will create, modify and delete VLANs for the entire VTP domain.

Answer: BC

Explanation/Reference:

考點：考查vtp的client模式特性，不能增加，刪除，修改vlan，只能從sever的報文中接收。

A：忽略VTP資訊，錯誤

B：轉發vtp資訊，正確

C：處理vtp資訊，正確

D：自己產生vtp通告，錯誤

E：創建，修改，刪除vlan，錯誤

QUESTION 266

A network administrator issues the ping 192.168.2.5 command and successfully tests connectivity to a host that has been newly connected to the network. Which protocols were used during the test? (Choose two.)(11.3.1-1)(9.7.1-1)

A. ARP

B. CDP

C. DHCP

D. DNS

E. ICMP

Answer: AE

Explanation/Reference:

考點：ping的資料包處理過程，涉及兩個協議。ARP,用於獲取目的mac位址；icmp，ping調用的協議。

QUESTION 267

Refer to the exhibit. Host A is to send data to Host B. How will Router1 handle the data frame received from Host A? (Choose three.)(1.4.5-2)

A. Router1 will strip off the source MAC address and replace it with the MAC address on the forwarding

FastEthernet interface.

B. Router1 will strip off the source IP address and replace it with the IP address on the forwarding

FastEthernet interface.

C. Router1 will strip off the destination MAC address and replace it with the MAC address of Host B.

D. Router1 will strip off the destination IP address and replace it with the IP address of Host B.

E. Router1 will forward the data frame out interface FastEthernet0/1.

F. Router1 will forward the data frame out interface FastEthernet0/2.

Answer: ACF

Explanation/Reference:

考點：考查乙太網環境中資料包從源到目的過程源/目的ip和mac位址的變化。源目IP不變，源目mac變

A：R1使用轉發介面的mac替換源MAC，正確

B：替換源IP，錯誤，源IP不變

C：替換目的IP為下一跳入介面的mac，正確

D:替換目的IP，錯誤

E：從f0/1轉發資料，錯誤，根據arp表應該是f0/2

QUESTION 268

Refer to the exhibit. What will Router1 do when it receives the data frame shown? (Choose three.)(1.4.5-2)

A. Router1 will strip off the source MAC address and replace it with the MAC address 0000.0c36.6965.

B. Router1 will strip off the source IP address and replace it with the IP address 192.168.40.1.

C. Router1 will strip off the destination MAC address and replace it with the MAC address 0000.0c07.4320.

D. Router1 will strip off the destination IP address and replace it with the IP address of 192.168.40.1.

E. Router1 will forward the data packet out interface FastEthernet0/1.

F. Router1 will forward the data packet out interface FastEthernet0/2.

Answer: ACF

Explanation/Reference:

考點：資料通信過程源目IP/mac位址的變化。

A：源MAC替換為f0/2口的mac，正確

B：替換源IP，錯誤

C：替換目的mac為下一跳入口的MAC，正確

D：替換目的IP，錯誤

E：從f0/1轉發資料，錯誤 F：正確

QUESTION 269

Refer to the exhibit. Host A pings Host B. What source MAC address and source IP address are contained in the frame as the frame leaves R2 destined for host B?(11.4.3-1)

A. abcd.abcd.a001

B. abcd.abcd.b002

C. abcd.abcd.c003

D. 10.2.0.15

E. 10.0.64.1

F. 10.0.128.15

Answer: BD

Explanation/Reference:

考查知識點和題目51一樣

QUESTION 270

Refer to the exhibit. A network associate has configured OSPF with the command:

City(config-router)# network 192.168.12.64 0.0.0.63 area 0 After completing the configuration, the associate discovers that not all the interfaces are participating in

OSPF. Which three of the interfaces shown in the exhibit will participate in OSPF according to this configuration statement? (Choose three.)(11.2.3-1)

A. FastEthernet0 /0

B. FastEthernet0 /1

C. Serial0/0

D. Serial0/1.102

E. Serial0/1.103

F. Serial0/1.104

Answer: BCD

Explanation/Reference:

考點，考查ospf network 命令中反遮罩的特性。192.168.12.64 0.0.0.63，相當於通告了192.168.12.64/26的網路，這個網路的範192.168.12.64.0--192.168.127.255/26綜上所述，BCD

QUESTION 271

A Catalyst 2950 needs to be reconfigured. What steps will ensure that the old configuration is erased?(Choose three.)(2.3.8-1)

A. Erase flash.

B. Restart the switch.

C. Delete the VLAN database.

D. Erase the running configuration.

E. Erase the startup configuration.

F. Modify the configuration register.

Answer: BCE

Explanation/Reference:

考點：考查恢復交換機出廠配置。有兩點，1、配置資訊，保存在startup-config中；2、vlan資料庫，存放在vlan.dat中。

A：刪除flash，錯誤，這樣把IOS都刪除了

B：重啟交換機，正確，刪除後重啟

C：刪除vlan資料庫，正確

QUESTION 272

Refer to the exhibit. The FMJ manufacturing company is concerned about unauthorized access to the Payroll Server. The Accounting1, CEO, Mgr1, and Mgr2 workstations should be the only computers withaccess to the Payroll Server. What two technologies should be implemented to help prevent unauthorized access to the server? (Choose two.)(3.1.1-2)(5.1.3-1)

A. access lists

B. encrypted router passwords

C. STP

D. VLANs

E. VTP

F. wireless LANs

Answer: AD

Explanation/Reference:

考點：考查ACL和VLAN的作用。

通過vlan隔離網段，通過acl實現控制一部分IP訪問

QUESTION 273

Refer to the exhibit. What commands must be configured on the 2950 switch and the router to allow communication between host 1 and host 2? (Choose two.)(6.2.2-1)

A. Router(config)# interface fastethernet 0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shut down

B. Router(config)# interface fastethernet 0/0

Router(config-if)# no shut down

Router(config)# interface fastethernet 0/0.1

Router(config-subif)# encapsulation dot1q 10

Router(config-subif)# ip address 192.168.10.1 255.255.255.0

Router(config-subif)# interface fastethernet 0/0.2

Router(config-subif)# encapsulation dot1q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

C. Router(config)# router eigrp 100

Router(config-router)# network 192.168.10.0

Router(config-router)# network 192.168.20.0

D. Switch1(config)# vlan database

Switch1(config-vlan)# vtp domain XYZ

Switch1(config-vlan)# vtp server

E. Switch1(config)# interface fastethernet 0/1

Switch1(config-if)# switchport mode trunk

F. Switch1(config)# interface vlan 1

Switch1(config-if)# ip default-gateway 192.168.1.1

Answer: BE

Explanation/Reference:

考點：考查單臂路由的配置。路由器上使用子介面，一個子介面對應一個vlan，封裝vlan id。交換機與路由器的互聯口使用trunk

QUESTION 274

Which three Layer 2 encapsulation types would be used on a WAN rather than a LAN? (Choose three.)(1.2.3-1)

A. HDLC

B. Ethernet

C. Token Ring

D. PPP

E. FDDI

F. Frame Relay

Answer: ADF

Explanation/Reference:

考點：考查廣域網鏈路的二層封裝協議

主要有HDLC,PPP,FR

QUESTION 275

Refer to the exhibit. The two connected ports on the switch are not turning orange or green. What would be the most effective steps to troubleshoot this physical layer problem? (Choose three.)(8.4.2-3)

A. Ensure that the Ethernet encapsulations match on the interconnected router and switch ports.

B. Ensure that cables A and B are straight-through cables.

C. Ensure cable A is plugged into a trunk port.

D. Ensure the switch has power.

E. Reboot all of the devices.

F. Reseat all cables.

Answer: BDF

Explanation/Reference:

考點：考查交換機埠顏色的定義。綠色正常，橘黃色有故障。

A：確認乙太網封裝是否匹配，錯誤。封裝錯誤不會在埠LED顯示

B：確認線纜A/B是否為直通線，正確。異構設備使用直通線

C：確認線纜A是否插到trunk口，錯誤。

D：確認是否通電，正確。

E：重啟設備，錯誤。

F：重置所有電纜，正確。

QUESTION 276

Refer to the exhibit. Which three statements are true about how router JAX will choose a path to the 10.1.3.0/24 network when different routing protocols are configured? (Choose three.)(9.3.1-1)(11.3.1-1)(11.3.1-3)

A. By default, if RIPv2 is the routing protocol, only the path JAX-ORL will be installed into the routing table.

B. The equal cost paths JAX-CHI-ORL and JAX- NY-ORL will be installed in the routing table if RIPv2 is the routing protocol.

C. When EIGRP is the routing protocol, only the path JAX-ORL will be installed in the routing table by default.

D. When EIGRP is the routing protocol, the equal cost paths JAX-CHI-ORL, and JAX-NY-ORL will be installed in the routing table by default.

E. With EIGRP and OSPF both running on the network with their default configurations, the EIGRP paths will be installed in the routing table.

F. The OSPF paths will be installed in the routing table, if EIGRP and OSPF are both running on the network with their default configurations.

Answer: ADE

Explanation/Reference:

本題還是路由選路的問題，首先我們要知道T1線路的速率為1.544M T3線路的速率為45M。RIP僅看 hop不關心鏈路狀態。eigrp和ospf等協議的度量都是跟路帶寬來計算的。所以要考察帶寬的大小。

QUESTION 277

Refer to the exhibit. Which three statements correctly describe Network Device A? (Choose three.)(6.4.1-1)(6.5.1-1)

A. With a network wide mask of 255.255.255.128, each interface does not require an IP address.

B. With a network wide mask of 255.255.255.128, each interface does require an IP address on a unique IP subnet.

C. With a network wide mask of 255.255.255.0, must be a Layer 2 device for the PCs to communicate With each other.

D. With a network wide mask of 255.255.255.0, must be a Layer 3 device for the PCs to communicate With each other.

E. With a network wide mask of 255.255.254.0, each interface does not require an IP address.

Answer: BDE

Explanation/Reference:

本題是子網問題。如果遮罩為255.255.255.128的話。那麼這個設備的兩個介面肯定要在不同的子網。因為10.1.0.36與10.1.1.70就屬於不同的子網。其實這題，主要考察的事遮罩的問題。如果想中間的設備不需要三層設備作路由那麼兩邊必須在同一個子網。需要在同一個子網。最小的遮罩就是255.255.254.0

QUESTION 278

Switch ports operating in which two roles will forward traffic according to the IEEE 802.1w standard?(Choose two.)(5.4.3-1)

A. alternate

B. backup

C. designated

D. disabled

E. root

Answer: CE

Explanation/Reference:

不管是802.1w還是802.1d等stp協議。用來轉發資料的介面都只有dp和rp

QUESTION 279

Refer to the exhibit. Given the output shown from this Cisco Catalyst 2950, what is the most likely reason that interface FastEthernet 0/10 is not the root port for VLAN 2?(5.2.4-5)(5.2.4-2)

A. This switch has more than one interface connected to the root network segment in VLAN 2.

B. This switch is running RSTP while the elected designated switch is running 802.1d Spanning Tree.

C. This switch interface has a higher path cost to the root bridge than another in the topology.

D. This switch has a lower bridge ID for VLAN 2 than the elected designated switch.

Answer: C

Explanation/Reference:

在選舉rp的時候。首先考慮的是到達跟橋cost最小的為rp，我個人認為在本交換機上對於vlan2也應該有一個rp從這圖我們看到有兩個root可以判斷他有可能使用的事pvst，那麼pvst對於每一個vlan每一個交換機應該有一個rp，只是我這個介面沒有本sw上另外一個介面到達rb的 cost小而已但這題的選項只有C更適合

QUESTION 280

Refer to the exhibit. The network administrator must establish a route by which London workstations canforward traffic to the Manchester workstations. What is the simplest way to accomplish this?(2.4.1-1)(2.4.3-2)

A. Configure a dynamic routing protocol on London to advertise all routes to Manchester.

B. Configure a dynamic routing protocol on London to advertise summarized routes to Manchester.

C. Configure a dynamic routing protocol on Manchester to advertise a default route to the London router.

D. Configure a static default route on London with a next hop of 10.1.1.1.

E. Configure a static route on London to direct all traffic destined for 172.16.0.0/22 to 10.1.1.2.

F. Configure Manchester to advertise a static default route to London.

Answer: E

Explanation/Reference:

london 要訪問manchester的主機，那麼我們只要知道那個一個靜態路由講資料包發出去就可以，通過子網匯總172.16.0.0/22可以代表manchester的3個網段。並且制定嚇一跳為10.1.1.2

QUESTION 281

Refer to the exhibit. This command is executed on 2960Switch:

2960Switch(config)# mac-address-table static 0000.00aa.aaaa vlan 10 interface

fa0/1 Which two of these statements correctly identify results of executing the command? (Choose two.)(2.3.6-4)

A. Port security is implemented on the fa0/1 interface.

B. MAC address 0000.00aa.aaaa does not need to be learned by this switch.

C. Only MAC address 0000.00aa.aaaa can source frames on the fa0/1 segment.

D. Frames with a Layer 2 source address of 0000.00aa.aaaa will be forwarded out fa0/1.

E. MAC address 0000.00aa.aaaa will be listed in the MAC address table for interface fa0/1 only.

Answer: BE

Explanation/Reference:

我們在一個介面靜態制定了mac位址的話。那麼這個介面指接搜這個mac位址的frame，並且mac位址會保存在mac add table 表明是static制定的

QUESTION 282

Which of the following describes the roles of devices in a WAN? (Choose three.)(2.1.4-2)(1.2.2-2)

A. A CSU/DSU terminates a digital local loop.

B. A modem terminates a digital local loop.

C. A CSU/DSU terminates an analog local loop.

D. A modem terminates an analog local loop.

E. A router is commonly considered a DTE device.

F. A router is commonly considered a DCE device.

Answer: ADE

Explanation/Reference:

csu/dsu是用來阻止數位環路的，而貓是用來阻止類比環路的。路由器是DTE設備。所以必須需要DCE設備來給其提供時鐘同步。

QUESTION 283

What are two characteristics of Telnet? (Choose two.)(4.2.4-2)

A. It sends data in clear text format.

B. It is no longer supported on Cisco network devices.

C. It is more secure than SSH.

D. It requires an enterprise license in order to be implemented.

E. It requires that the destination device be configured to support Telnet connections.

Answer: AE

Explanation/Reference:

telnet在資料傳遞的時候是明文的。不夠安全。並且如果要對設備使用telnet我們要在vty線路下配置密碼，並且要允許telnet連接到設備

QUESTION 284

What are two security appliances that can be installed in a network? (Choose two.)(4.1.4-2)

A. ATM

B. IDS

C. IOS

D. IOX

E. IPS

F. SDM

Answer: BE

Explanation/Reference:

B.入侵檢測系統

E.入侵防禦系統

QUESTION 285

A single 802.11g access point has been configured and installed in the center of a square office. A few wireless users are experiencing slow performance and drops while most users are operating at peak efficiency. What are three likely causes of this problem? (Choose three.)(7.4.2-1)(7.4.1-1)

A. mismatched TKIP encryption

B. null SSID

C. cordless phones

D. mismatched SSID

E. metal file cabinets

F. antenna type or direction

Answer: CEF

Explanation/Reference:

能夠對802.11g有干擾的，應該都是工作在2.4G左右的設備。比如說微波爐，無繩電話，金屬對於無線是有吸附作用的。可能會引起信號的衰竭

QUESTION 286

A host is attempting to send data to another host on a different network. What is the first action that the sending host will take?(9.7.1-1)(9.7.2-1)

A. Drop the data.

B. Send the data frames to the default gateway.

C. Create an ARP request to get a MAC address for the receiving host.

D. Send a TCP SYN and wait for the SYN ACK with the IP address of the receiving host.

Answer: C

Explanation/Reference:

主機要發送資料的話。首先要請求到閘道或者下一跳的mac位址進行二層的封裝

QUESTION 287

Refer to the exhibit. What two facts can be determined from the WLAN diagram? (Choose two.)(7.1.4-2)

A. The area of overlap of the two cells represents a basic service set (BSS).

B. The network diagram represents an extended service set (ESS).

C. Access points in each cell must be configured to use channel 1.

D. The area of overlap must be less than 10% of the area to ensure connectivity.

E. The two APs should be configured to operate on different channels.

Answer: BE

Explanation/Reference:

QUESTION 288

Which two devices can interfere with the operation of a wireless network because they operate on similar frequencies? (Choose two.) (7.4.1-1)

A. copier

B. microwave oven

C. toaster

D. cordless phone

E. IP phone

F. AM radio

Answer: BD

Explanation/Reference:

能夠對wireless 802.11g有干擾的，應該都是工作在2.4G左右的設備。比如說微波爐，無繩電話，金屬對於無線是有吸附作用的。可能會引起信號的衰竭

QUESTION 289

Which two statements describe characteristics of IPv6 unicast addressing? (Choose two.)(7.3.2-2)

A. Global addresses start with 2000::/3.

B. Link-local addresses start with FE00:/12.

C. Link-local addresses start with FF00::/10.

D. There is only one loopback address and it is ::1.

E. If a global address is assigned to an interface, then that is the only allowable address for the interface.

Answer: AD

Explanation/Reference:

ipv6的常規問題，globaladdress的其實段位2000::/3這個是定義好了的,D選項說的是換回地址,也就是ipv4的127.0.0.1這個類似的.所以D正確

QUESTION 290

Refer to the exhibit. Which statement is true?(5.3.4-2)

A. The Fa0/11 role confirms that SwitchA is the root bridge for VLAN 20.

B. VLAN 20 is running the Per VLAN Spanning Tree Protocol.

C. The MAC address of the root bridge is 0017.596d.1580.

D. SwitchA is not the root bridge, because not all of the interface roles are designated.

Answer: D

Explanation/Reference:

stp對於RB所有的介面都是dp

QUESTION 293

Refer to the exhibit. A system administrator installed a new switch using a script to configure it. IP connectivity was tested using pings to SwitchB. Later attempts to access NewSwitch using Telnet fromSwitchA failed. Which statement is true?(5.4.9-3)(8.4.3-5)

A. Executing password recovery is required.

B. The virtual terminal lines are misconfigured.

C. Use Telnet to connect to RouterA and then to NewSwitch to correct the error.

D. Power cycle of NewSwitch will return it to a default configuration.

Answer: C

Explanation/Reference:

B.虛擬終端（VTY）配置不正確

QUESTION 294

Which two of these statements regarding RSTP are correct? (Choose two.)(5.4.3-1)(5.4.1-1)

A. RSTP cannot operate with PVST+.

B. RSTP defines new port roles.

C. RSTP defines no new port states.

D. RSTP is a proprietary implementation of IEEE 802.1D STP.

E. RSTP is compatible with the original IEEE 802.1D STP.

Answer: BE

Explanation/Reference:

RSTP定義了新的埠類型，包括隔離埠，備份埠等，rstp如果與stp同時存在的話。那麼將相容成stp

QUESTION 295

Which three of these statements regarding 802.1Q trunking are correct? (Choose three.)(3.1.2-1)(3.2.1-3)(3.2.1-4)

A. 802.1Q native VLAN frames are untagged by default.

B. 802.1Q trunking ports can also be secure ports.

C. 802.1Q trunks can use 10 Mb/s Ethernet interfaces.

D. 802.1Q trunks require full-duplex, point-to-point connectivity.

E. 802.1Q trunks should have native VLANs that are the same at both ends.

Answer: ACE

Explanation/Reference:

802.1q native vlan是不做標記的，並且在trunk兩邊的native需要一致

QUESTION 296

Refer to the exhibit. Each of these four switches has been configured with a hostname, as well as being configured to run RSTP. No other configuration changes have been made. Which three of these show the correct RSTP port roles for the indicated switches and interfaces? (Choose three.)(5.3.4-1)

A. SwitchA, Fa0/2, designated

B. SwitchA, Fa0/1, root

C. SwitchB, Gi0/2, root

D. SwitchB, Gi0/1, designated

E. SwitchC, Fa0/2, root

F. SwitchD, Gi0/2, root

Answer: ABF

Explanation/Reference:

stp的問題，我們可以通過mac位址確定跟橋為switchC，那麼C上的埠都為dp，A的f0/1，d的g0/2為rp，那麼B哪一個是RP呢。從D過來的bpdu的cost都為23，從a過來的為19+19=38，所以B的rp為gi0/1，那麼阻塞埠應該在A和B之間的網段，A的f0/2這個網段收到的BPDUcost最小為19，而B的g0/2收到的BPDUcost最小應該為23，所以B的 g0/2應為BLK.1G鏈路的cost為4,100M鏈路cost為19.100M鏈路cost為19.1G鏈路cost為4.

QUESTION 297

Refer to the exhibit. A junior network administrator was given the task of configuring port security on SwitchA to allow only PC_A to access the switched network through port fa0/1. If any other device is detected, the port is to drop frames from this device. The administrator configured the interface and tested it with successful pings from PC_A to RouterA, and then observes the output from these two show commands.

Which two of these changes are necessary for SwitchA to meet the requirements? (Choose two.)(2.4.6-1)

A. Port security needs to be globally enabled.

B. Port security needs to be enabled on the interface.

C. Port security needs to be configured to shut down the interface in the event of a violation.

D. Port security needs to be configured to allow only one learned MAC address.

E. Port security interface counters need to be cleared before using the show command.

F. The port security configuration needs to be saved to NVRAM before it can become active.

Answer: BD

Explanation/Reference:

題目要求，只允許PCA在f0/1口，那麼我們首先要設置port-security，並且mac最大為1，而且還要是用

sticky

來記錄pc-a的mac，這樣就可以達到f0/1只能為pc-a使用

QUESTION 298

Which of the following correctly describe steps in the OSI data encapsulation process? (Choose two.)(4.1.1-1)(5.1.1-1)

A. The transport layer divides a data stream into segments and may add reliability and flow control information.

B. The data link layer adds physical source and destination addresses and an FCS to the segment.

C. Packets are created when the network layer encapsulates a frame with source and destination host addresses and protocol-related control information.

D. Packets are created when the network layer adds Layer 3 addresses and control information to a segment.

E. The presentation layer translates bits into voltages for transmission across the physical link.

Answer: AD

Explanation/Reference:

傳輸層將資料分段，並且加入flow control等資訊。在網路層就將資料進行打包，並加上ip包頭，鏈路層。將包進行成Frame，封裝二層頭部

QUESTION 299

For which type of connection should a straight-through cable be used?(10.2.2-4)

A. switch to switch

B. switch to hub

C. switch to router

D. hub to hub

E. router to PC

Answer: C

Explanation/Reference:

直通線連接在不同的設備之上，所以這裏我們選擇不同的設備之間的連接

QUESTION 300

Which set of commands is recommended to prevent the use of a hub in the access layer?(2.4.6-1)(2.4.6-2)

A. switch(config-if)#switchport mode trunk

switch(config-if)#switchport port-security maximum 1

B. switch(config-if)#switchport mode trunk

switch(config-if)#switchport port-security mac-address 1

C. switch(config-if)#switchport mode access

switch(config-if)#switchport port-security maximum 1

D. switch(config-if)#switchport mode access

switch(config-if)#switchport port-security mac-address 1

Answer: C

Explanation/Reference:

說怎麼解決接入層接hub的問題，防止介入hub可以使用port-security限制埠的mac位址數量就解決了