QUESTION 401

Which of the following describes the roles of devices in a WAN? (Choose three.)(2.1.4-1)(1.2.2-1)

A. A CSU/DSU terminates a digital local loop

B. A modem terminates a digital local loop

C. A CSU/DSU terminates an analog local loop

D. A modem terminates an analog local loop

E. A router is commonly considered a DTE device

F. A router is commonly considered a DCE device

Answer: ADE

Explanation/Reference:

QUESTION 402

How should a router that is being used in a Frame Relay network be configured to avoid split horizon issues from preventing routing updates?(3.3.1-2)

A. Configure a separate sub-interface for each PVC with a unique DLCI and subnet assigned to the subinterface

B. Configure each Frame Relay circuit as a point-to-point line to support multicast and broadcast traffic

C. Configure many sub-interfaces on the same subnet

D. Configure a single sub-interface to establish multiple PVC connections to multiple remote router interfaces

Answer: A

Explanation/Reference:

QUESTION 403

The Frame Relay network in the diagram is not functioning properly. What is the cause of the problem?(3.2.2-1)

A. The Gallant router has the wrong LMI type configured

B. Inverse ARP is providing the wrong PVC information to the Gallant router

C. The S3 interface of the Steele router has been configured with the frame-relay encapsulation ietf command

D. The frame-relay map statement in the Attalla router for the PVC to Steele is not correct

E. The IP address on the serial interface of the Attalla router is configured incorrectly

Answer: D

Explanation/Reference:

QUESTION 404

As a CCNA candidate, you must have a firm understanding of the IPv6 address structure. Refer to IPv6 address, could you tell me how many bits are included in each filed?(7.3.2-2)

A. 24

B. 4

C. 3

D. 16

Answer: D

Explanation/Reference:

QUESTION 405

Refer to the exhibit. How many broadcast domains exist in the exhibited topology?(3.1.1-3v)

A. one

B. two

C. three

D. four

E. five

F. six

Answer: C

Explanation/Reference:

QUESTION 406

Refer to the exhibit.

The network administrator has created a new VLAN on Switch1 and added host C and host D. The administrator has properly configured switch interfaces FastEthernet0/13 through FastEthernet0/24 to be members of the new VLAN. However, after the network administrator completed the configuration, host A

could communicate with host B, but host A could not communicate with host C or host D. Which commands are required to resolve this problem?(6.2.2-1)

A. Router(config)# interface fastethernet 0/1.3

Router(config-if)# encapsulation dot1q 3

Router(config-if)# ip address 192.168.3.1 255.255.255.0

B. Router(config)# router rip

Router(config-router)# network 192.168.1.0

Router(config-router)# network 192.168.2.0

Router(config-router)# network 192.168.3.0

C. Switch1# vlan database

Switch1(vlan)# vtp v2-mode

Switch1(vlan)# vtp domain cisco

Switch1(vlan)# vtp server

D. Switch1(config)# interface fastethernet 0/1

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# switchport trunk encapsulation isl

Answer: A

Explanation/Reference:

QUESTION 407

On a network of one department, there are four PCs connected to a switch, as shown in the following figure: After the Switch1 restarts. Host A ( the host on the left ) sends the first frame to Host C (the host on the right). What the first thing should the switch do?(2.2.1-1)

A. Switch1 will add 192.168.23.12 to the switching table.

B. Switch1 will add 192.168.23.4 to the switching table.

C. Switch1 will add 000A.8A47.E612 to the switching table.

D. None of the above

Answer: C

Explanation/Reference:

QUESTION 408

Refer to the exhibit. The network administrator is in a campus building distant from Building B. WANRouter is hosting a newly installed WAN link on interface S0/0. The new link is not functioning and the administrator needs to determine if the correct cable has been attached to the S0/0 interface. How can the administrator accurately verify the correct cable type on S0/0 in the most efficient manner?(8.1.2-1)

A. Telnet to WANRouter and execute the command show interfaces S0/0

B. Telnet to WANRouter and execute the command show processes S0/0

C. Telnet to WANRouter and execute the command show running-configuration

D. Telnet to WANRouter and execute the command show controller S0/0

E. Physically examine the cable between WANRouter S0/0 and the DCE.

F. Establish a console session on WANRouter and execute the command show interfaces S0/0

Answer: D

Explanation/Reference:

QUESTION 409

While troubleshooting a connectivity issue from a PC you obtain the following information:

Local PC IP address: 10.0.0.35/24

Default Gateway: 10.0.0.1

Remote Sever: 10.5.75.250/24

You then conduct the following tests from the local PC:

Ping 127.0.0.1 – Successful

Ping 10.0.0.35 – Successful

Ping 10.0.0.1 – Unsuccessful

Ping 10.5.75.250 – Unsuccessful

What is the underlying cause of this problem?(11.3.5-2)

A. A remote physical layer problem exists.

B. The host NIC is not functioning.

C. TCP/IP has not been correctly installed on the host.

D. A local physical layer problem exists.

Answer: D

Explanation/Reference:

QUESTION 410

A network administrator is troubleshooting the OSPF configuration of routers R1 and R2. The routerscannot establish an adjacency relationship on their common Ethernet link. The graphic shows the output ofthe show ip ospf interface e0 command for routers R1 and R2. Based on the information in the graphic, what is the cause of this problem?(11.2.5-2)

A. The OSPF area is not configured properly.

B. The priority on R1 should be set higher.

C. The cost on R1 should be set higher.

D. The hello and dead timers are not configured properly.

E. A backup designated router needs to be added to the network.

F. The OSPF process ID numbers must match.

Answer: D

Explanation/Reference:

QUESTION 411

This graphic shows the results of an attempt to open a Telnet connection to router ACCESS1 from router Remote27. Which of the following command sequences will correct this problem?(11.2.2-2)

A. ACCESS1(config)# line console 0

ACCESS1(config-line)# password cisco

B. Remote27(config)# line console 0

Remote27(config-line)# login

Remote27(config-line)# password cisco

C. ACCESS1(config)# line vty 0 4

ACCESS1(config-line)# login

ACCESS1(config-line)# password cisco

D. Remote27(config)# line vty 0 4

Remote27(config-line)# login

Remote27(config-line)# password cisco

E. ACCESS1(config)# enable password cisco

F. Remote27(config)# enable password cisco

Answer: C

Explanation/Reference:

QUESTION 412

When upgrading the IOS image, the network administrator receives the exhibited error message. What could be the cause of this error?(4.5.4-2)(4.5.3-1)(11.2.3-2)

A. The new IOS image is too large for the router flash memory.

B. The TFTP server is unreachable from the router.

C. The new IOS image is not correct for this router platform.

D. The IOS image on the TFTP server is corrupt.

E. There is not enough disk space on the TFTP server for the IOS image.

Answer: B

Explanation/Reference:

QUESTION 413

Which statement is true?(7.3.2-1)

A. An IPv6 address is 64 b long and is represented as hexadecimal characters.

B. An IPv6 address is 32 b long and is represented as decimal digits.

C. An IPv6 address is 128 b long and is represented as decimal digits.

D. An IPv6 address is 128 b long and is represented as hexadecimal characters.

Answer: D

Explanation/Reference:

IPV6地址長度為128比特,十六進位

QUESTION 414

Which command sets and automatically encrypts the privileged enable mode password?(11.2.2-2)

A. enbale password c1sco

B. secret enable c1sco

C. password enable c1sco

D. enable secret c1sco

Answer: D

Explanation/Reference:

A和D是設置密碼命令，D是加密密碼設置

QUESTION 415

In which circumstance are multiple copies of the same unicast frame likely to be transmitted in a switched LAN?(7.2.7-1)

A. after broken links are re-established

B. in an improperly implemented redundant topology

C. when upper-layer protocols require high reliability

D. during high traffic periods

E. when a dual ring topology is in use

Answer: E

Explanation/Reference:

題目問在哪一個環境下交換機在lan出現多個相同的單播幀

A.鏈路斷後重啟

B.拓撲中有不合適的冗餘

C上層協定需要高可靠度

D在高流量的階段

E在雙環拓撲，正確

QUESTION 416

Refer to the exhibit.Which is the most efficient summarization that R1 can use to advertise its networks to R2?(2.6.1-1)

A. 172.1.4.0/22

B. 172.1.0.0/21

C. 172.1.0.0/22

D. 172.1.4.0/25

172.1.4.128/25

172.1.5.0/24

172.1.6.0/24

172.1.7.0/24

E. 172.1.4.0/24

172.1.5.0/24

172.1.6.0/24

172.1.7.0/24

Answer: A

Explanation/Reference:

做一個最精確的路由匯總，將圖中IP地址做匯總

QUESTION 417

Refer to the exhibit.Based on the exhibited routing table,how will packets from a host within the 192.168.10.192/26 LAN be forwarded to 192.168.10.1?(9.2.6-3)

A. The router will forward packets from R3 to R2 to R1

B. The router will forward packets from R3 to R1

C. The router will forward packets from R3 to R1 to R2

D. The router will forward packets from R3 to R2 to R1 AND from R3 to R1

Answer: D

Explanation/Reference:

從路由表中可以看到，到目的地址有兩條等價均衡的路徑

QUESTION 418

Which layer 2 protocol encapsulation type supports synchronous and asynchronous circuis and has built-in security mechanisms?(2.2.2-1)

A. Frame Relay

B. HDLC

C. x.25

D. PPP

Answer: D

Explanation/Reference:

哪一個二層協定封裝支援同步和非同步電路並內嵌安全機制？

答案：PPP

QUESTION 419

Which command can be used to verify the DLCI destination address in a Frame Relay static configuration?(3.2.2-1)

A. show frame-relay end-to-end

B. show frame-relay map

C. show frame-relay lmi

D. show frame-relay pvc

Answer: B

Explanation/Reference:

QUESTION 420

Refer to the exhibit.The network administrator requires easy configuration options and minimal routing protocol traffic.Which two options provide adequate routing table information for traffic that passes between the two routers and satisfy the requests of the network administrator?(choose two)(2.6.2-1)

A. a dynamic routing procotol on InternetRouter to advertise summarized routers to CentralRouter.

B. a dynamic routing procotol on CentralRouter to advertise summarized routers to InternetRouter.

C. a static route on InternetRouter to direct traffic that is destined for 172.16.0.0/16 to CentralRouter.

D. a dynamic routing procotol on InternetRouter to advertise all routes to CentralRouer.

E. a dynamic routing procotol on CentralRouer to advertise all routes to InternetRouter

F. a static,default route on CentralRouter that directs traffic to InternetRouter.

Answer: CF

Explanation/Reference:

題目要求用簡單設置和最小路由流量的路由，所以採用靜態路由或默認路由

QUESTION 421

Given an IP address 172.16.28.252 with a subnet mask of 255.255.240.0, what is the correct network addres?(6.2.2-1)

A. 172.16.16.0

B. 172.16.24.0

C. 172.16.0.0

D. 172.16.28.0

Answer: A

Explanation/Reference:

求網段的網路位址，即此網段的第一個IP位址

QUESTION 422

Refer to the exibit. Three cisco 2950 switches are set to their default priority settings.During the spanningtree process,which switch will be elected as the root bridge?(5.2.1-3)

A. Switch3

B. Switch4

C. Switch2

D. Switch1

Answer: D

Explanation/Reference:

默認優先順序，看mac地址，最小的選為跟橋

QUESTION 423

Which three elements must be used when you configure a router interface for vlan trunking?(choose three)(6.1.1-3)

A. one IP network or subnetwork for each subinterface

B. subinterface numbering that matches vlan tages

C. subinterface encapsulation identifiers that match vlan tags

D. a management domain for each subinterface

E. one physical interface for each subinterface

F. one subinterface per vlan

Answer: ACF

Explanation/Reference:

考察單臂路由的設置

A一個網路或子網對應一個子介面，正確

B子介面號必須和vlan號一樣，不正確

C子介面封裝識別字必須和vlan號一樣，正確

D每一個子介面需要一個管理域，不正確

E每一個子介面需要一個物理埠，不正確

F一個子介面對應一個vlan，正確

QUESTION 424

A receiving host computes the checksum on a frame and determines that the frame is damaged.The frame is then discarded.At which OSI layer did this happen?(7.1.1-1)

A. session

B. network

C. physical

D. data link

E. transport

Answer: D

Explanation/Reference:

幀的處理是在二層

QUESTION 425

A router receives information about network 192.168.10.0/24 from multiple sources.What will the router consider the most reliable information about the path to that network?(3.4.1-1)

A. an OSPF update for network 192.168.0.0/16

B. a static router to network 192.168.10.0/24

C. a static router to network 192.168.10.0/24 with a local serial interface configured as the next hop

D. a RIP update for network 192.168.10.0/24

E. a directly connected interface with an address of 192.168.10.254/24

F. a default route with a next hop address of 192.168.10.1

Answer: E

Explanation/Reference:

直連埠的路由管理距離最小為0

QUESTION 426

Which two statements describe the advantages to the use of RIP over the use of OSPF?(choose two)(11.0.1-1)

A. RIP requires less time to converge

B. RIP uses less bandwidth

C. RIP is less complex to configure

D. RIP demands fewer router resources

E. RIP has a more accurate metric

Answer: CD

Explanation/Reference:

RIP相比OSPF的好處？

A RIP彙聚時間更短。錯

BRIP佔用更少的帶寬。錯。距離向量協議採用定時更新，占帶寬較大

CRIP 設置更簡單，正確

DRIP佔用更少的路由器資源，正確

ERIP有一個更精確的metric，錯，使用跳數作為metric，不精確

QUESTION 427

Which statement about vlan operation on cisco catalyst switches is true?(3.1.1-3)

A. when a packet is received from an 802.1Q trunk,the vlan id can be determined from the source MAC address table.

B. unkown unicast frames are retransmitted only to the ports that belong to the same vlan

C. ports between switches should be configured in access mode so that vlans can span across the ports

D. broadcast and multicast frames are retransmitted to ports that are configured on different vlan.

Answer: B

Explanation/Reference:

A通過源mac位址確定vlan，不正確

B未知單播幀只在本vlan傳播，正確

C交換機之間應該設置為trunk

Dvlan能分割廣播域，廣播不能跨vlan傳輸

QUESTION 428

Which commands are required to properly configure a router to properly configure a router to run OSPF and to add network 192.168.16.0/24 to OSPF area 0?(choose two)(11.2.3-1)

A. Router(config)#router ospf 1

B. Router(config)#router ospf 0

C. Router(config)#router ospf area 0

D. Router(config-router)#network 192.168.16.0 0.0.0.255 area 0

E. Router(config-router)#network 192.168.16.0 0.0.0.255 0

F. Router(config-router)#network 192.168.16.0 255.255.255.0 area 0

Explanation/Reference:

OSPF的配置

進程號範圍1-65535，宣告網段需要反遮罩+區域

QUESTION 429

The enable secret command is used to secure access to which CLI mode?(11.2.3-3)

A. user EXEC mode

B. global configuration mode

C. privileged EXEC mode

D. auxiliary setup mode

Answer: C

Explanation/Reference:

注意此題不是問在什麼模式下設置，而是問為進入那種模式保障安全

QUESTION 430

Refer to the exhibit.Explain how the routes in the table are being affected by the status change on interface Ethernet0.(5.3.3-1)

A. The router is poisoning the routes and multicasting the new path costs via interface Ethernet1.

B. The router is receiving updates about unreachable networks from router that are connected to interface Ethernet1.

C. The router is poisoning the routes and broadcasting the new path costs via interface Ethernet1.

D. The router is requesting updates for these networks from routers that are connected to interface Etherne1.

Answer: A

Explanation/Reference:

E0埠down，部分路由收不到更新，將此部分路由的metric設為16，通過E1埠發佈

QUESTION 431

In which integration method is an IPV6 packet encapsulated within an IPV4 protocol?(7.3.3-1)

A. proxy.

B. dual-stack

C. tunneling

D. dot1q

Answer: C

Explanation/Reference:

將IPV6包封裝在IPV4協議內，隧道技術

QUESTION 432

Which two commands can be used to verify a trunk link configurationt status on a given cisco switch?(choose two)(3.3.3-1)(3.4.1-3)

A. show interfaces interface

B. show interfaces interface trunk

C. show interfaces interface switchport

D. show ip interface brief

E. show interfaces vlan

Answer: BC

Explanation/Reference:

查看trunk命令，show interface trunk和show interface switchport ，只能選BC

QUESTION 433

Assuming the default switch configuration,which approach should you use to configure the extended vlan range(1006 through 4094) on a cisco catalyst 3750 series switch?(3.1.1-4)

A. Configure the switch to be in VTP client mode.

B. Configure the switch to be in VTP domaint mode.

C. Configure the switch to be in VTP transparent mode.

D. Configure the switch to be in VTPv2.

Answer: C

Explanation/Reference:

vlan 號1006-4094屬於擴展vlan，只能在透明模式配置

QUESTION 434

http://certiking.blogspot.com/

http://certiking.blogspot.com/

You are working in a data center environment and are assigned the address range 10.188.31.0/23.You are asked to develop an IP addressing plan to allow the maximum number of subnets with as many as 30 hosts each.Which IP address range meets these requirements?(6.5.3-1)

A. 10.188.31.0/27

B. 10.188.31.0/26

C. 10.188.31.0/29

D. 10.188.31.0/28

E. 10.188.31.0/25

Answer: B

Explanation/Reference:

30台主機還需要一個閘道，所以需要31個有效IP位址

QUESTION 435

Which three statements accurately describe layer 2 Ethernet switches?(choose three)(4.0.1-1)(3.1.1-3)(6.1.2-1)

A. Microsegmentation decreases the number of collisions on the network.

B. if a switch receives a frame for an unkown destination,it uses ARP to resolve the address.

C. Spanning Tree Protocol allows switches to automatically share vlan information.

D. In a properly functioning network with redundant switched paths,each switched aegment will contain one root bridge with all its ports in the forwarding state.All other switches in that broadcast domain will haveonly one root port.

E. Establishing vlans increases the number of broadcast domains.

F. Switches that are configured with vlans make forwarding decisions based on both layer 2 and layer 3 address information.

Answer: ADF

Explanation/Reference:

A為分割減少衝突的數量，這裏是衝突並不是衝突域，正確

B收到未知目的幀會產生泛洪

C這是vtp的特性，不是stp的

D在一個能夠正常運行的網路冗餘交換路徑，每一個交換網段將包括一個跟橋。所有埠都在轉發狀態，其他本廣播域的交換機有一個根埠。正確

E.不看題目要求這句話沒問題，但是題目要求對二層的描述，廣播域就不和題意了

F vlan使轉發基於二層和三層，正確

QUESTION 436

Refer to the exhibit.The Lakside Company has the internetwork in the exhibit.The administrator would like to reduce the size of routing table on the Central router.Which partial routing table entry in the Central route represents a route summary that represents the vlans in Phoenix but no additional subnets?(8.1.4-4)

A. 10.0.0.0/22 is subnetted,1 subnets

D 10.4.0.0[90/20514560] via 10.2.0.2,6w0d,serial0/1

B. 10.0.0.0/28 is subnetted,1 subnets

D 10.2.0.0[90/20514560] via 10.2.0.2,6w0d,serial0/1

C. 10.0.0.0/22 is subnetted,1 subnets

D 10.0.0.0[90/20514560] via 10.2.0.2,6w0d,serial0/1

D. 10.0.0.0/30 is subnetted,1 subnets

D 10.2.2.0[90/20514560] via 10.2.0.2,6w0d,serial0/1

E. 10.0.0.0/28 is subnetted,1 subnets

D 10.4.4.0[90/20514560] via 10.2.0.2,6w0d,serial0/1

F. 10.0.0.0/30 is subnetted,1 subnets

D 10.4.4.4[90/20514560] via 10.2.0.2,6w0d,serial0/1

Answer: A

Explanation/Reference:

對PHOENIX做路由匯總

QUESTION 437

Refer to the exhibit.Which two devices can be used to complete the connection between the WAN router at the customer site and the service provider?(2.1.4-1)(2.1.3-1)

A. modem

B. Frame Relay switch

C. CSU/DSU

D. WAN switch

E. ISDN TA

F. ATM switch

Answer: AC

Explanation/Reference:

問號處應該是DCE設備.A和C是DCE設備，一個是類比的一個是數位的

Which PPP subprotocol negotiates authentication options?(2.2.2-2)

A. NCP

B. LCP

C. ISDN

D. DLCI

E. SLIP

Answer: B

Explanation/Reference:

PPP的哪一個子協議能夠協商驗證：LCP

QUESTION 439

Which two data link layers are supported by cisco ios software for IPv6?(choose two)(7.3.1-3)(7.3.4-1)

A. PPP

B. FDDI

C. Frame Relay PVC

D. NBMA

E. Frame Relay SVC

Answer: AC

Explanation/Reference:

支援IPV6的二層協定有哪些

QUESTION 440

Which term describes a spanning-tree network that has all switch ports in either the blocking or forwarding state?(5.2.5-1)

A. redundant

B. spanned

C. provisioned

D. converged

Answer: D

Explanation/Reference:

收斂以後交換機埠只有堵塞和轉發狀態

QUESTION 441

Which two of these are characteristics of the 802.1Q protocol?(choose two)

A. it is a layer 2 messaging protocol which maintains vlan configurations across network.(3.2.1-3)

B. it includes an 8-bit field which specifies the priority of a frame.

C. it is used exclusively for tagging vlan frames and dose not address network reconvergence following switched network topology changes.

D. it modifies the 802.3 frame header,and thus requires that the FCS be recomputed.

E. it is a trunking protocol capable of carring untagged frames

Answer: DE

Explanation/Reference:

A描述的是vtp的特性

B應該是3bit

Cexclusively是唯一的意思。802.1Q不是唯一的

D修改幀的頭部，需要重新進行FCS校驗，正確

E是一個trunk協定並支援不打標記的幀，正確

QUESTION 442

Refer to the exhibit.Which two statements are true of the interfaces on switch1?(choose two)(2.3.4-1)

A. A hub is connected directly to FastEthernet0/5

B. FastEthernet0/1 is configured as a trunk link.

C. FastEthernet0/5 has statically assigned mac address

D. Interface FastEthernet0/2 has been disable.

E. Multiple devices are connected directly to FastEthernet0/1.

F. FastEthernet0/1 is connected to a host with multiple network interface cards.

Answer: AB

Explanation/Reference:

圖中可以看出F0/1,F0/2為trunk。F0/5埠有多個mac位址，而且不是trunk，只能是接了一個hub

QUESTION 443

Which two statements apply to dynamic access lists?(choose two)(5.4.2-1)

A. they offer simpler management in large internetworks.

B. you can control logging messages.

C. they allow packets to be filtered based on upper-layer session information.

D. you can set a time-based security policy.

E. they provide a level of security against spoofing.

F. they are used to authenticate individual users.

Answer: CD

Explanation/Reference:

考察動態控制列表

A 在大的網路提供簡單的管理。沒有這功能，錯誤

B 控制logging 資訊。錯誤

C 允許包過濾基於上層會話，正確

D 可以設置基於時間的安全機制，正確

E 提更級別安全防欺騙攻擊，不正確

F 用於鑒定個人用戶，不準確，如果選三項，可以考慮F

QUESTION 444

A network engineer wants to allow a temporary entry for a remote user with a specific usename and password so that the user can access the entire network over the internet.which ACL can be used?(5.4.2-1)

A. reflexive

B. extended

C. standard

D. dynamic

Answer: D

Explanation/Reference:

網路工程師想要允許一個臨時站與一個特定的usename遠端用戶和密碼,這樣用戶可以訪問整個網路.whichACL可以使用?動態ACL

QUESTION 445

Which parameter or parameters are used to calculate OSPF cost in cisco routers?(11.3.1-1)

A. Bandwidth,Delay,and MTU

B. Bandwidth

C. Bandwidth and MTU

D. Bandwidth , MTU,Reliability,Delay,and Load

Answer: B

Explanation/Reference:

OSPF使用帶寬計算cost

QUESTION 446

which command is used to enable CHAP authentication whit PAP as the fallback method on a serial interface? (2.4.5-1)

A. (config-if)#authentication ppp chap fallback ppp

B. (config-if)#authentication ppp chap pap

C. (config-if)#ppp authentication chap pap

D. (config-if)#ppp authentication chap fallback ppp

Answer: C

Explanation/Reference:

要求使用chap認證，pap認證作為備用

QUESTION 447

assuming the default switch configuration which vlan range can be added modified and removed on a cisco switch?(3.1.1-4)

A. 2 through 1001

B. 1 through 1001

C. 1 through 1002

D. 2 through 1005

Answer: A

Explanation/Reference:

交換機上默認那些是可以修改和移動的vlan。2-1001

QUESTION 448

which address range efficiently summarizes the routing table of the addresses for router main?(2.6.1-1)

A. 172.16.0.0/18

B. 172.16.0.0/16

C. 172.16.0.0/20

D. 172.16.0.0/21

Answer: C

Explanation

將這11個IP地址做匯總

QUESTION 449

What is the most likely reason for the dispartly between the actual network numbers at the branches and the routes in the routing table on Gateway-Router?(5.3.1-1)(7.4.2-1)

A. Branch-Router2 is configured to send both RIPv1 and RIPv2 updates.

B. Gateway-Router is configured to only receive RIPv2 updates.

C. Gateway-router is configured to receive only RIPv1 updates.

D. Branch-Router1 is configured to only send RIPv1 updates

Answer: D

Explanation/Reference:

RIPV1是有類路由，路由表中可以看出R1的路由是主類的

QUESTION 450

A network administrator needs to configure port security on a switch.which two statements are true?(choose two)(2.4.6-2)(2.3.8-1)

A. The network administrator can apply port security to dynamic access ports

B. The network administrator can configure static secure or sticky secure mac addresses in the voice vlan.

C. The sticky learning feature allows the addition of dynamically learned addresses to the running configuration.

D. The network administrator can apply port security to EtherChannels.

E. When dynamic mac address learning is enabled on an interface,the switch can learn new addresses,up to the maximum defined.

Answer: CE

Explanation/Reference:

A.動態埠不能應用的埠安全

B.in the vioce vlan不正確

C.保存在running configuratin 正確

D.EtherChannels 不能應用埠安全

E自動學習mac位址可以學到所設置的最大數量，正確

QUESTION 451

The network technician is planning to use the 255.255.255.224 subent mask on the network.which three valid IP addresses can the technician use for the hosts?(choose three)(6.5.1-3)

A. 172.22.243.127

B. 172.22.243.191

C. 172.22.243.190

D. 10.16.33.98

E. 10.17.64.34

F. 192.168.1.160

Answer: CDE

Explanation/Reference:

塊大小是32，所以最後一位是32的倍數或32倍數-1都不可以

QUESTION 452

Which command helps a network administrator to manage memory by displaying flash memory and NVRAM utilization?(4.5.2-1)

A. show secure

B. show file systems

C. show flash

D. show version

Answer: B

Explanation/Reference:

查看flash和NVRAM的裏利用率

QUESTION 453

Which IEEE standard protocol is initiated as a result of successful DTP completion in a switch over FastEthernet?(5.4.3-1)

A. 802.3ad

B. 802.1w

C. 802.1Q

D. 802.1d

Answer: C

Explanation/Reference:

DTP: 資料傳送協定

QUESTION 454

Which command enhances the 802.1D convergence time on ports that are connected to hosts?(5.2.5-3)

A. spanning-tree backbonefast

B. spanning-tree uplinkfast

C. spanning-tree portfast

D. spanning-tree cost512

Answer: C

Explanation/Reference:

連接到主機的埠使用spanning-tree portfast

QUESTION 455

Which two statements are characteristics of a distance vector routing protocol?(choose two)(4.1.2-2)(4.1.1-1)

A. RIP is an example of distance vector routing protocols.

B. Updates are periodic and include the entire routing table.

C. Routing updates are sent only after topology changes

D. The protocol can be useful in hub-and-spoke and hierarchical networks.

E. Convergence is usually faster than with link state protocols

F. Each router has its own view of the topology

Answer: AB

Explanation/Reference:

A RIP是距離向量路由協議

B 週期性更新整個路由表

C 這條是鏈路狀態協定的特性

D 協定可以用在中心型分層網路

E 會聚比鏈路狀態要慢

F 只是和鄰居更新，不會維護整個拓撲

QUESTION 456

Which of the following protocols uses both TCP and UDP ports?(3.3.1-1)

A. SMTP

B. Telnet

C. FTP

D. DNS

Answer: D

Explanation/Reference:

主機使用功能變數名稱解析用的是UDP。功能變數名稱解析伺服器之間同步資訊使用的是TCP.

QUESTION 457

Which two privileged mode cisco ios commands can be used to determine a cisco router chassis serial number?(choose two)(11.1.6-2)(4.5.2-2)

A. show inventory

B. show flash:filesys

C. dir flash:|include chassis

D. show diag

E. show controllers

Answer: AD

Explanation/Reference:

考察查看機器序列號的命令

QUESTION 458

Refer to the exhibit.A new subnet with 60 hosts has been added to the network.Which subnet address should this network use to provide enough usable addresses while wasting the fewest addresses?(6.5.1-3)

A. 192.168.1.56/27

B. 192.168.1.64/26

C. 192.168.1.64/27

D. 192.168.1.56/26

Answer: B

Explanation/Reference:

要考慮到圖中已經用掉的網段

QUESTION 459

Refer to the exhibit.Why are two OSPF designated routers identified on Core-Router?(11.4.3-1)

A. Core-Router is connected more than one multi-access network

B. The router at 208.149.23.130 is a secondary DR in case the primary fails.

C. Two router IDs have the same OSPF priority and are therefore tied for DR election

D. The DR ecection is still underway and there are two contenders for the role.

Answer: A

Explanation/Reference:

為什麼一個路由器上會出現倆DR?

DR/BDR是指埠並不是指路由器，所以一個路由器接入多個ospf網路，會出現此種情況

QUESTION 460

It has become necessary to configure an existing serial interface to accept a second Frame Relay virtual circuit.Which of the following processes are required to accomplish this task?(choose three)(3.4.1-1)

A. configure static frame relay map entries for each subinterface network.

B. remove the ip address from the physical interface

C. create the virtual interfaces with the interface command

D. configure each subinterface with its own IP address

E. disable split horizon to prevent routing loops between the subinterface networks

F. encapsulate the physical interface with multipoint PPP

Answer: BCD

Explanation/Reference:

現存有一條幀中繼需要加第二條幀中繼，需要創建子介面，使用點對多點

A 點對多點需要frame-relay map

E取消水準分割不是為了防止路由環路

F用多點PPP封裝物理埠,錯

QUESTION 461

Refer to the graphic.Computer 1 is consoles into switch A.Telnet connections and pings run from the command prompt on switch A fail.Which of the following could cause this problem?(2.3.6-1)

A. switch A dose not have a cdp entry for switch B or router JAX

B. switch A dose not have an IP address

C. port 1 on switch A should be an access port rather than a trunk port

D. switch A is not directly connected to router JAX

E. switch A does not have a default gateway assigned

Answer: E

Explanation/Reference:

圖中vlan1沒有配置IP位址，這可能管理vlan不是vlan1，別的vlan沒有顯示出來。

題目說ping和telnet都失敗，至少說明交換機已經配置了IP位址。原因就是沒有配置默認閘道了

QUESTION 462

Refer to the exhibit.What is the result of setting the no login command?(11.2.2-2)

A. The is a virtually limitless supply of IP addresses

B. Telnet access requires a new password at first login

C. Telnet access requires a password

D. Telnet access is denied

Answer: A

Explanation/Reference:

以前此題的答案是D，不正確。選項A的意思也有點模糊，但是BCD是肯定不對，已經實驗驗證。

QUESTION 463

Which IPv6 address is valid?(7.3.2-1)

A. 2031:0:130F::9C0:876A:130B

B. 2001:0DB8:0000:130F:0000:0000:08GC:140B

C. 2001:0DB8:0:130H::87C:140B

D. 2031::130F::9C0:876A:130B

Answer: A

Explanation/Reference:

B十六進位沒有G

C也沒有H

D 只能有一個：：

Refer to the exhibit.Which three variables(router,protocol port,and router ACL direction)apply to an extended ACL that will prevent student 01 from securely browsing the internet?(5.1.8-1)(5.1.4-1)

A. OUT

B. Router 3

C. HTTTP

D. IN

E. Router 1

Answer: BCD

Explanation/Reference:

我們選擇在R3上的in方向。如果在R3的out的方向會增加路由器不必要的負擔

QUESTION 465

What are two characteristics of a switch that is configured as a VTP client?(choose two)(4.2.4-1)(4.2.3-2)

A. if a switch that is configured to operate in client mode cannot access a VTP server,then the switch reverts to transparent mode.

B. The local vlan configuration is updated only when an update that has a higher configuration revision number is received.

C. Vtp advertisements are not forwarded to neighboring switches that are configured in vtp transparent mode.

D. When switches in vtp client mode are rebooted,they send a vtp advertisement request to the vtp servers.

E. Vtp client is the default vtp mode

F. On switches that are configured to operate in client mode ,vlans can be created,deleted,or renamed Locally

Answer: BD

Explanation/Reference:

考察VTP 用戶端的特點

A 如果交換機在用戶端模式不能介入VTP伺服器，將會轉到透明模式。錯，不會轉到透明模式

B 只有當收到更高的修正版本號更新，本地vlan設置才會更新。正確

C 透明模式下VTP廣播不會轉發給鄰居路由器。錯，可以轉發

D客戶模式下重啟，發送一個vtp請求。正確

E 默認模式應該是伺服器模式

F用戶端模式不能修改創建vlan

QUESTION 466

What two statistics appear in show frame-relay map output?(choose two)(3.1.2-2)

A. The number of FECN packets that are received by the router

B. The number of BECN packets that are received by the router.

C. The ip address of the local router

D. The value of the local DLCI

E. The status of the PVC that is configured on the router

Answer: DE

Explanation/Reference:

QUESTION 467

Which command can you use to determine the cisco ios feature set on a cisco router?(11.1.6-2)

A. show version

B. dir flash:|include ios

C. show environment

D. show diag

E. show inventory

Answer: A

Explanation/Reference:

查看路由器IOS特性

QUESTION 468

What are three benefits of implementing vlans?(choose three)(3.1.1-3)

A. A more efficient use of bandwidth can be achieved allowing many physical groups to use the same network infrastructure

B. Broadcast storms can be mitigated by decreasing the number of broadcast domains,thus increasing their size.

C. A higher level of network security can be reached by separating sensitive data traffic from other network traffic.

D. Port-based vlans increase switch-port use efficient,thanks to 802.1Q trunks

E. A more efficient use of bandwidth can be achieved allowing many logical networks to use the same network infrastructure.

F. Broadcast storms can be mitigated by increasing the number of broadcast domains,thus reducing their size.

G. VLANs make it easier for IT staff to configure new logical groups,because the vlans all belong to the same broadcast domain.

Answer: CEF

Explanation/Reference:

A錯，不是通過物理上組合，而是邏輯上的

B錯，應該是增加廣播域的數量

C一個更高的層次網路安全可以實現,以分離的敏感資料從其他網路。正確

D thanks to 802.1Q trunks不正確

E正確，logical是邏輯的意思

F正確

G錯誤，是VTP的作用不是vlan的

QUESTION 469

In the implementation of VLSM techniques on a network using a single Class C IP address,which subnet mask is the most efficient for point-to-point serial links?(6.5.1-1)

A. 255.255.255.240

B. 255.255.255.254

C. 255.255.255.252

D. 255.255.255.0

E. 255.255.255.248

Answer: C

Explanation/Reference:

點到點鏈路需要兩個有效IP位址

QUESTION 470

A switch has been configured with two vlans and is connected to a router with a trunk for inter-vlan routing. OSPF has been configured on the router,as the routing protocol for the network.Which statement about this network is true?(3.4.4-1)

A. For the two vlans to communicate,a network statement for the trunk interface needs to be added to the OSPF configuration.

B. For the two vlans to communicate,a network statement for each subinterface needs to be added to the OSPF configuration.

C. Direct inter-vlan communication does not require OSPF.

D. OSPF cannot be used if router-on-a-stick is configured on the router.

Answer: C

Explanation/Reference:

兩個vlan之間通信使用的是直連路由，不需要OSPF

QUESTION 471

Which command enables IPv6 forwarding on a cisco router?(7.3.7-1)

A. ipv6 host

B. ipv6 unicast-routing

C. ipv6 local

D. ipv6 neighbor

Answer: B

Explanation/Reference:

在路由器上啟動IPV6的命令

QUESTION 472

Router A is unable to reach Router B.Both routers are running ios version 12.0.After reviewing the command output and graphic,what is the most likely cause of the problem?(11.2.1-1)

（此題暫時沒圖）

A. incorrect bandwidth configuration

B. incorrect LMI configuration

C. incorrect map statement

D. incorrect IP address

Answer: C

Explanation/Reference:

QUESTION 473

A cisco router is booting and has just completed the POST process.It is now ready to find and load an IOSimage.what function does the router perform next?(4.5.7-2)

A. It checks the configuration register

B. It attempts to boot from a TFTP server

C. It loads the first image file in flash memory

D. It inspects the configuration file in NVRAM for boot instructions

Answer: A

Explanation/Reference:

下一步檢查configuration register 然後決定載入配置

QUESTION 474

Refer to the exhibit. Which statement describes DLCI 17?(3.1.5-1)

A. DLCI 17 describes the ISDN circuit between R2 and R3.

B. DLCI 17 describes a PVC on R2. It cannot be used on R3 or R1.

C. DLCI 17 is the Layer 2 address used by R2 to describe a PVC to R3.

D. DLCI 17 describes the dial-up circuit from R2 and R3 to the service provider.

Answer: C

Explanation/Reference:

DLCI號是一個二層位址，和mac類似

QUESTION 475

Which two subnetworks would be included in the summarized address of 172.31.80.0 /20? (Choose two)(6.4.1-1)(6.5.1-2)

A. 172.31.17.4/30

B. 172.31.51.16/30

C. 172.31.64.0/18

D. 172.31.80.0/22

E. 172.31.92.0/22

F. 172.31.192.0/18

Answer: DE

Explanation/Reference:

172.31.80.0----172.31.95.255

QUESTION 476

Which spread spectrum technology does the 802.11b standard define for

operation?(7.1.2-1)

A. IR

B. DSSS

C. FHSS

D. DSSS and FHSS

E. IR, FHSS, and DSSS

Answer: B

Explanation/Reference:

802.11b採用DSSS擴頻技術

QUESTION 477

Refer to the exhibit.HostA cannot ping HostB.Assuming routing is properly configured,what could be the cause of this problem?(6.4.1-1)

A. HostA is not on the same subnet as its default gateway

B. The address of SwitchA is a subnet address

C. The Fa0/0 interface on RouterA is on a subnet that cannot be used

D. The serial interfaces of the routers are not on the same subnet

E. The Fa0/0 interface on RouterB is using a broadcast address

Answer: D

Explanation/Reference:

QUESTION 478

Which protocol provides a method of sharing VLAN configuration information between two Cisco switch?(4.1.1-2)

A. VTP

B. 802.1Q

C. RSTP

D. STP

Answer: A

Explanation/Reference:

交換機共用vlan資訊，VTP協定

QUESTION 479

Which three approaches can be used while migrating from an IPV4 addressing scheme to an IPV6 scheme. (choose three)(7.3.3-1)

A. statically map IPV4 address to IPV6 addresses

B. configuration IPv4 tunnels between IPV6 islands

C. use DHCPv6 to map IPV4 addresses to IPV6 addresses

D. use proxying and translation to translate IPV6 packets into IPV4 packets

E. configure IPV6 directly

F. enable dual-stack routing

Answer: ABF

Explanation/Reference:

答案是ABF。IPV6向IPV4過渡書上有三種方法：隧道，雙棧，NAT-PT.

A是靜態NAT-PY

B是隧道

F是雙棧

QUESTION 480

DNS servers provide what service?(3.3.1-2)

A. They run a spell check on host names to ensure accurate routing

B. They convert domain names into IP address

C. Given an IP address,they determine the name of the host that is sought

D. They map individual hosts to their specific IP addresses

Answer: B

Explanation/Reference:

功能變數名稱解析伺服器將網址功能變數名稱解析為IP地址

QUESTION 481

What is the alternative notation for the IPV6 address B514:82C3:0000:0000:0029:EC7A:0000:EC72?(7.4.2-1)

A. B514:82C3:0029::EC7A:0000:EC72

B. B514:82C3:0029:EC7A:EC72

C. B514:82C3::0029:EC7A:0:EC72

D. B514:82C3::0029:EC7A:EC72

Answer: C

Explanation/Reference:

考試時候C可能會出現14:82C3::0029:EC7A:0:EC72這種情況，系統錯誤，不用管它，選C

QUESTION 482

which command allows you to verify the encapsulation type (CISCO OR IETF) for a frame relay link?(3.1.5-1)(3.2.1-2)

A. show frame-relay map

B. show frame-relay lmi

C. show inter serial

D. show frame-relay pvc

Answer: A

Explanation/Reference:

幀中繼封裝有兩種cisco和ietf。默認是cisco。通過命令show frame-relay可以查看

QUESTION 483

the network administrator has been asked to give reasons for moving from ipv4 to ipv6. what are two valid reasons for adopting ipv6 over ipv4?(choose two)(7.3.1-3)

A. telnet access does not require a password

B. nat

C. no broadcast

D. chanage of destination address in the ipv6 header

E. chanage of source address in the ipv6 header

F. autoconfiguration

Answer: CF

Explanation/Reference:

要求給出IPV4向IPv6轉換的理由，也就是v6的優點。C沒有廣播, F自動設置

QUESTION 484

Which IPsec security protocol should be used when confidentiality is required?(6.3.7-1)

A. PSK

B. AH

C. MD5

D. ESP

Answer: D

Explanation/Reference:

ESP 提供機密性、資料源認證、無連接的完整性、抗重播服務（一種部分序列完整性的形式） 和有限資訊流機密性。

QUESTION 485

At which layer of the OSI model is RSTP used to prevent loops?(5.4.3-1)

A. data link

B. network

C. physical

D. transport

Answer: A

Explanation/Reference:

QUESTION 486

Which device might be installed at a branch office to enable and manage an IPsec site-to-site VPN?(6.3.2-1)

A. Cisco IOS IPsec/SSL VPN client

B. Cisco VPN Clinet

C. ISDN terminal adapter

D. Cisco Adaptive Security Appliance

Answer: D

Explanation/Reference:

QUESTION 487

Which statement is true, as relates to classful or classless routing?(7.0.1-1)

A. Automatic summarization at classful boundries can cause problems on discontinuous subnets

B. EIGRP and OSPF are classful routing protocols and summarize routes by default

C. RIPv1 and OSPF are classless routing protocols

D. Classful routing protocols send the subnet mask in routing updates

Answer: A

Explanation/Reference:

A 自動匯總將路由匯總成主類位址，在不連續網段會出錯。正確

B錯都是無類路由協議 OSPF沒有自動匯總

C 錯 RIPV1是有類路由協議

D錯有類路由協定路由更新不需要子網路遮罩

QUESTION 488

Refer to the exhibit, Host A pings interface S0/0 on router 3, what is the TTL value for that ping?(5.1.7.1)(11.3.1-1)(4.4.7-1)

A. 253

B. 252

C. 255

D. 254

Answer: A

Explanation/Reference:

默認是255，每經過一個路由器減1

QUESTION 489

Which address is the IPV6 all-RIP-routers muticast group address that is used by RIPng as the destaion address?(7.3.6-2)

A. FF02::A

B. FF05::101

C. FF02::9

D. FF02::6

Answer: C

Explanation/Reference:

IPV6 RIPng的組播地址 FF02::9

QUESTION 490

What is the default administrative distance of OSPF?(11.1.7-1)

A. 120

B. 100

C. 90

D. 110

Answer: D

Explanation/Reference:

OSPF默認管理距離110

QUESTION 491

Which encapsulation type is a Frame Relay encapsulation type that is supported by Cisco routers?(3.2.1-2)(3.1.5-1)

A. IETF

B. HDLC

C. Q9333-A Annex A

D. ANSI Annex D

Answer: A

Explanation/Reference:

A. Cisco 支援Frame-Relay封裝格式 : Cisco & IETF

B. HDLC -> Cisco 預設 point to point 封裝

C & D -> 是 Frame-Relay lmi-type

QUESTION 492

Which two statements about using the CHAP authentication mechanism in a PPP link are true?(choose two)(2.4.1-1)(2.4.5-2)

A. CHAP uses a two-way handshake

B. CHAP authentication periodically occurs after link establishment

C. CHAP has no protection from playback attacks

D. CHAP authentication is performed only upon link establishment

E. CHAP uses a three-way handshake

F. CHAP authentication passwords are sent in plaintext

Answer: BE

Explanation/Reference:

PAP是兩次握手機制；CHAP通過3次握手，CHAP定期發出挑戰，以確保遠端節點有有效的密碼，CHAP采用MD5加密演算法，而PAP採用明文

A.CHAP認證採用的是兩次握手機制

B.CHAP的認證是鏈路建立之後週期性發生的

C.CHAP認證沒有重放攻擊的保護

D.執行CHAP認證只是在鏈路建立的時候

E.CHAP認證採用的三次握手機制

F.CHAP認證的密碼是採用明文的方式傳輸的

QUESTION 493

Refer to the exhibit.What is the reason that the interface status is “administratively down,line protocol down”?(8.4.2-2)

A. There is a mismatch in encapsulation types

B. The wrong type of cable is connected to the interface

C. The interface has been configured with the shutdown command

D. The interface is not receiving any keepalives

E. The interface needs to be configured as a DTE device

F. There is no encapsulation type configured

Answer: C

Explanation/Reference:

注釋：很明顯是物理層的問題，以前題庫中出現過此題，以前是雙選，現在變成單選了。仔細看一下B選項被捨棄了，因為這是串口，串口線只有一種兩個埠，DTE和DCE，兩頭接錯屬於時鐘配置錯誤是protocol down。

QUESTION 494

Which command encrypts all plaintext passwords?(11.2.2-2)

A. Router(config)#password-encryption

B. Router(config)#service password-encryption

C. Router#service password-encryption

D. Router# password-encryption

Answer: B

Explanation/Reference:

QUESTION 495

Which Cisco IOS diagnostics command can disrupt the operation of a router under high-load conditions?(4.5.6-5)

A. show running-config

B. show processes cpu

C. debug all

D. logging host ip address

Answer: C

Explanation/Reference:

題目說的了路由器是在高負載的情況下運行的，而debug命令是要佔用大量的CPU的佔用率很高的，debug不要亂用，資料流程量很大的時候，debug資訊會把記憶體全部消耗，造成設備死機。

QUESTION 496

Which three statements about EIGRP are true?(choose three)(9.1.5-1)(9.4.6-1)(4.0.1-1)(4.1.4-1)

A. EIGRP converges fast RIP because of DUAL and backup routes that are stored in the topology table.

B. EIGRP uses a hello protocol to establish neighbor relationships

C. EIGRP uses split horizon and reverse poisoning to avoid routing loops.

D. EIGRP uses periodic updates to exchange routing information

E. EIGRP allows routers of different manufacturers to interoperate

F. EIGRP supports VLSM and authentication for routing updates.

G. EIGRP use a broadcast address to send routing information.

Answer: ABF

Explanation/Reference:

QUESTION 497

Refer to the exhibit.Why dose the telnet connectin fail when a host attempts to connect a remote router?(4.2.4-2)

A. No password was set for tty lines

B. No password was set for aux lines

C. No password was set for vty lines

D. No password was set for cty lines

Answer: C

Explanation/Reference:

QUESTION 498

Refer to the exhibit,The VLAN configuration of S1 is not being in this VTP enabled environment.The VTPand uplink port configurations for each switch are displayed.Which two command sets,if issued,resolve this failure and allow VTP to operate as expected?(choose two)(4.2.4-1)(4.3.1-3)(3.3.4-1)

A. S2(config)#vtp mode transparent

B. S1(config)#vtp mode client

C. S2(config) #interface f0/24

S2(config-if) #switchport mode access

S2(config-if) #end

D. S2(config)#vtp mode client

E. S1 (config)#interface f0/24

S1(config-if)#switchport mode trunk

S1(config-if)#end

Answer: BE

Explanation/Reference:

如圖所示，SW1的VLAN配置在此VTP下沒有被啟用，VTP和它的上行埠配置如圖所示，請問使用哪兩個命令可以排除此故障，達到VTP預期的效果？SW1和SW2都處於同一個VTP模式下，但SW1處於透明模式，僅轉發通告，地整個域內的資訊不能同步；SW1是接入模式，SW2是動態自動模式，二者結合是接入模式，所以應該將SW1配置為中繼模式，VTP域中的各個VLAN才能進行通信。

A. S2(config)#vtp mode transparent //將s2 vtp的模式設為透明模式

B. S1(config)#vtp mode client //將s1 vtp的模式設為客服端模式

C. S2(config) #interface f0/24 //進入s2的0/24好埠

S2(config-if) #switchport mode access //將其設為可訪問模式

S2(config-if) #end //退出埠配置模式

D. S2(config)#vtp mode client //將s2 VTP的模式設為客服端模式

E. S1(config)#interface f0/24 //進入S1的0/24號埠

S1(config-if)#switchport mode trunk //將其設為幹到模式

S1(config-if)#end //退出埠配置模式